# Attachment 1

# 4.b. Description of Aircraft System

- Physical Characteristics.  A160 Hummingbird is a vertical takeoff and landing (VTOL), long endurance UAS that is 35 ft long with a 36 ft rotor diameter and a potential maximum gross weight of 6500 lbs.  Propulsion of the A160T is with a Pratt & Whitney PW207D turboshaft engine.  This engine is FAA certified, Type Certificate Data Sheet E42NE, dated April 29, 2002.

- Flight Performance.  The A160 cruises at 50-100 kts and can reach a maximum level speed of 150 kts.

- Method of pilotage.  The A160 uses a global positioning system (GPS) to navigate using Ground Control Station (GCS) operator input routing or pre-programmed mission plans.  The GCS provides real-time command, flight monitoring and mission re-tasking to the UAS.  The GCS operator console provides for manual routing of the vehicle.  The UAS operator can respond in real-time to ATC instructions in the same manner as manned aircraft.  Communication with the aircraft is provided by dual-redundant, two-way datalinks operating in the Ku-band frequency range.

- Launch and recovery methods.  The A160 Hummingbird takeoff and landing is commanded from the GCS.  It performs takeoffs and landings similar to standard, manned rotorcraft.

- Route and altitude procedures.  After launch, the A160 follows a preprogrammed waypoint mission plan or follows position and velocity command inputs by an operator in the GCS.