ATTACHMENT 8

7.c. EMERGENCY PROCEDURES

1.  INTRODUCTION

As a novel developmental aircraft, the A160 may experience in-flight emergencies due to a system failures that warrant special landing procedures.   The impact of these failures can be reduced with good failure identification tools, emergency landing procedures, emergency landing software, including autorotation control laws, and adequate flight crew training in emergency procedures.

The timeline for an emergency landing begins with identification of the failure.  A160 operators have many tools at their disposal to quickly recognize critical problems, including extensive telemetry and special-purpose displays.   The “Emergency Identification” section more fully describes these tools.   After identifying a failure, the pilot in command is responsible for deciding the proper course of action.   If an emergency landing is warranted, the pilot must choose the type of emergency landing, be it an expedited return home, an off-site landing, or an autorotation.  The pilot will then execute the appropriate procedures to achieve this.   The characteristics and procedures associated with each type of landing are described in the “Emergency Procedures” section.   The final section is dedicated to autorotation.  It describes the energy management problem, simulation of autorotation and current A160 autorotation control laws.

2.  EMERGENCY IDENTIFICATION

A160 operating teams have many tools at their disposal to identify emergencies.   They benefit from over 300 channels of telemetry, varying in update rate from 1 Hz to 1000 Hz.   These channels include roughly 30 strain gauges, 40 temperature sensors, 15 accelerometers, 10 pressure sensors, 20 current and voltage sensors, 20 engine sensors, 20 navigation sensors, 20 sensors dedicated to actuators, and various communications health signals.   All data are available to pilots and operators in real time and multiple detailed system-monitoring displays are available at designated engineering stations.   The key displays are the pilots' head up display (HUD), several engineering displays, and analog and digital stripcharts.

Figure 1 shows a screenshot of the head up display.   The most critical parameters for key subsystems are listed at the bottom of the window.   Each parameter is highlighted in color in if it is outside of pre-specified bounds, yellow for marginal or red for serious.  During normal flight, no items are highlighted, so the appearance of out-of-bounds signals is obvious.  In addition to highlighted numeric displays, the HUD includes a text warning box in the center bottom of the screen.   Text warnings are triggered for many more criteria than are available in the HUD numeric displays.   During normal flight, no text warnings are displayed, so the appearance of such warnings is clear evidence of circumstances to monitor more precisely.

Figure 1: Pilot's Head Up Display Screenshot (typical)

Figure 2 shows a screenshot of the engineering station Health Monitoring Display, a comprehensive searchable numeric display of every telemetry signal, including over 300 channels varying in update rate from 1 Hz to 1000 Hz.  These include roughly 30 strain gauges, 40 temperature sensors, 15 accelerometers, 10 pressure sensors, 20 current and voltage sensors, 20 engine sensors, 20 navigation sensors, 20 actuator sensors, and various communications health signals.  Parameter values that are outside of pre-determined safe limits are highlighted, yellow for marginal or red for serious.  Since this screen continuously displays all telemetry data, a more comprehensive picture of overall vehicle health is available, including subsystem details if an emergency arises, allowing assessment of potential emergency procedures.  It also supports vehicle health projections based on signal values and trends.

Figure 2: Health Monitoring Display Screenshot (typical)

The most flexible display is SigView, shown in Figure 3.   SigView provides real-time-configurable time history, cross-plot, FFT, gauge, slider, and numeric displays of any telemetry signal with configurable time scale and data smoothing.   The team configures a display appropriate to the flight’s objectives before takeoff, allowing detailed monitoring of test-specific failure modes.   The team may add displays during flight to investigate anomalies.

Figure 3: SigView Screenshot (typical)

Three analog stripcharts and one digital stripchart are also available in a typical flight test.  The stripcharts are used to examine the dynamic behavior of critical signals, such as speeds, loads, and temperatures.  Previous stripcharts are kept in the ground control station so that previous and current behavior can be compared.

Flight Crew Qualifications, Training,  and Simulator Practice

Valid engineering judgments by skilled and well-trained teams play a prominent role in emergency operations.  The A160 flight-test team engineers have extensive ground testing experience and most have been involved in the design and development of the A160 flight systems.   This provides an in-depth understanding of the entire system and facilitates the rapid identification, evaluation, and reaction to a system anomaly and potential emergency situation.

The flight crews have been trained in several ways.  Initially, the crew members have trained in flight operations on an A160 flight simulator.  Additional hours have been logged in the past with the Maverick UAS, an unmanned modification of the Robinson R-22 aircraft.  Before flying the A160, flight crew are trained in the co-pilot seat under supervision of a qualified A160 pilot.

3.   EMERGENCY PROCEDURES

Once the need for emergency landing is identified, the pilot in command must decide what type of landing to execute.  The pilot has the option of expediting a standard landing that returns to the launch site, landing off-site, or if there is a drive system, engine, or tail rotor control failure, commanding an autorotation.  This section describes each landing and the associated procedures.  During flight, pilots have access to detailed checklists for each procedure.

## 3.1 Expedited Landing

A160 emergency procedures recommend that the operator expedite a standard landing when it is not prudent to continue flight, but when losing the aircraft is not imminent.  Examples of this situation are slowly rising system temperatures that trend towards exceeding limits or the loss of a non-flight critical sensor like a torque sensor or exhaust gas temperature sensor.   In such a case, the pilot begins executing the standard landing procedure immediately, while initiating focused attention on the anomaly.  Standard landings approach the heli-pad at Lakehurst NAES (NEL), from the west (assuming normal winds) at 40 kts and 200 ft/min.  Below is the checklist for standard landings:

MANUAL LANDING

- Call to Tower:  Inbound to Land	COMPLETE

- Call Ground Crew:  Inbound to Land	COMPLETE

- RPM	SET FOR APPROACH

- Map Landing Location	SET

- “Takeoff Preset” horsepower (250 hp)	SET

- Altimeter	RESET

- Wind Compensation	OFF

- Wind Speed and  Direction	CHECK

- Payload Mode (if landing by EO/IR)	FIXED

- Altitude	 DESCEND TO 450 MSL (350 AGL)

- Airspeed		40 KTS

- Landing Gear	DOWN

- Landing Switch	LANDING

- FLIGHT PHASE: LANDING	CONFIRM

- Approach Path	SET

- At 6000’ range	450 MSL (350 AGL)

- Descent Rate	200 FPM

- Landing Gear	CONFIRM DOWN

- Landing Area	CLEAR

- Weight on Wheels Warning	CHECK

- Cross Landing Threshold	50 FEET

- FLIGHT PHASE: TAXI	CONFIRM ON GROUND CONTACT

- Squat Switch	ACTIVATE IF NO TAXI

- Landing Switch	DISENGAGE

## 3.2  Off-site Landing

If there is an immediate risk of losing control of the aircraft, A160 emergency procedures recommend that the pilot execute an off-site landing.  Examples of this situation are unexpected increases in vibrations or loss of generators requiring running on batteries for a lengthy period.  In an off-site landing, the pilot brings the A160 to a hover over a safe landing spot at an altitude high enough to maintain the telemetry link.  The pilot then engages the “abort” autonomous flight plan which executes a 200 ft/min hover descent and touchdown, even if the datalink is lost.  If link is still lost, the A160 after touchdown automatically shuts down the engine, fuel pumps, and cooling fans.

A sight survey was conducted of the Lakehurst NAES terrain to determine safe off-site landing locations.  A map of the safest off-site landing spots is shown in Figure 4, which is posted in the ground control station.  The A160 will operate within the lightly shaded region bounded by the bold red lines.  The operational boundary lines were chosen to stay within the 5 mile class D radius around Lakehurst NAES and to avoid over-flying major roadways that surround the airfield.

Below is the checklist for off-site landing descent:

OFF SITE LANDING

- Airspeed		40 KTS

- Descent Rate	200 FPM

- RPM	SET FOR APPROACH

- Wind Compensation	OFF

- “Takeoff Preset” horsepower (300 hp)	SET

- Landing Gear	DOWN

- Landing Switch	LANDING

- FLIGHT PHASE: LANDING	CONFIRM

- Approach Path and Landing Site	SET

- At ~200 feet AGL	SLOW TO HOVER

- Abort Switch	ENGAGE

- Call to Tower:  "Landing off site"	COMPLETE

- Call to Ground Crew:  "Landing off site"	COMPLETE

--------------------------------------------------------------------------------------------

Likely loss of datalink descending below 100 MSL

“Abort” automation will shut off aircraft after touchdown

--------------------------------------------------------------------------------------------

- Aircraft Location	RECORD

- Locate Aircraft	COMPLETE

- Safe and Shutdown Aircraft	COMPLETE

Figure 4: A160 Operating Area and Landing Zones

Primary Zone of Operations

Safest offsite landing sites

Airfield tower

Primary

Takeoff and Landing Site

## 3.3  Autorotation

In the case of engine or other drive system failure, the A160 must execute an autorotation.  By virtue of the A160 RPM control architecture, the A160 automatically enters and maintains autorotation upon loss of drive torque.  The pilot must then enable the touchdown logic using a panel switch, identify a safe landing location, and guide the aircraft to that location.  The autorotation timeline below provides a summary of autorotation events.  .

- Loss of power

- A160 automatically enters and maintains autorotative descent

- Operator enables autorotation switch.

  - A160 autonomously controls RPM to 280

  - Disallows lost link flight plan

  - Operator maintains control of speed and heading

    - Commands efficient airspeed ~60 kts

    - Navigates toward safe landing spot

- At ~200 ftAGL

  - A160 autonomously begins increasing rotor RPM to 405 to store maximum energy

  - A160 autonomously starts cyclic flare

- At ~100 ftAGL, A160 autonomously zeros heading rate and sideslip

- At ~30 ftAGL, A160 autonomously starts collective flare

- Just after touchdown, A160 autonomously shuts down engine, fuel pumps and cooling fans.

Below is the checklist for autorotation:

AUTOROTATION

- Forward Speed to 60 kts (if advisable)	SET

- Autorotation Switch	ENGAGE

- Landing Site	PICK

  - Steering is still operator controlled

  - Forward speed can be adjusted down to change glide path

After touchdown:

- Call to Tower:  Off site landing	COMPLETE

- Call Ground Crew: Off site landing	COMPLETE

The brevity of the checklist is meant to allow completion in the limited time available before the A160 reaches the ground.  To this end, the touchdown logic enabled by the autorotation switch automates several tasks, such as lowering the landing gear.