Attachment 6

7.a Lost Link Procedure

1.  General Lost Link Sequence

The Lost Link Plan is a pre-programmed flight plan that is loaded onto the aircraft computer when it is first powered up before a flight.  It is part of the Pre-flight Checklist.

The A160 is constantly receiving, and monitoring the health of, uplink messages from the GCS.  If the A160 does not receive any uplink from the ground control station (GCS) for 15 seconds, the A160 will automatically start its autonomous "lost link" plan.  The GCS will receive notice from the data link that it is not completing the UAS link.  While flying the preprogrammed "lost link" trajectory, the A160 and the GCS will continue to attempt to reestablish the link

The lost link flight plan instructs the A160 to fly straight back to the takeoff spot’s GPS location while descending to 200 ft AGL, and then execute a hover landing descent (see figure 1).  The flight is at 40 kts, with a hover landing descent of 130 fpm.  60 seconds after touchdown, the A160 will automatically shut the aircraft engine off.

ATC will be informed of the lost-link situation and that the A160 is inbound to land autonomously at the takeoff location without operator input.  Outside observers will continue to track and report on aircraft position until landing.

2.  Flight in Class D airspace of Lakehurst Naval Air Engineering Station

When in the Class D airspace at NEL, the A160 will follow the flight path described in Figure 1

Figure 1  Class D Airspace Operating Area of Lakehurst NAES (KNEL)

Lightly shaded region indicates area of operation and bold red lines indicate do not cross operational boundaries

Takeoff and landing spot marked with red square on Eastern side of area

Lost Link Approach to takeoff and landing spot is a straight line flight path from the operating point where link was lost