# Attachment 5

# 5.h. Launch/Recovery

- Launch and recovery methods.  The A160 Hummingbird will be towed or taxi from U.S. Army CERDEC Flight Activity Hangar 5 to takeoff location (see Figure).  Takeoff and landing is commanded from the ground control station and performs takeoffs and landings similar to standard, manned rotorcraft.  Launch only takes place after a successful completion of the UAS safety checklist.

- Method of pilotage.  The A160 uses a global positioning system (GPS) to navigate, using operator input routing or pre-programmed mission plans.  Communication with the aircraft is provided by dual-redundant, two-way datalinks.  The primary system operates in the Ku-band frequency range, an alternative dual system operates  in the C-band frequency range

- Route and altitude procedures.  After launch, the A160 follows a preprogrammed waypoint mission plan or follows position and velocity command inputs by an operator in the ground control station.

Figure:  Hangar 5 and launch area

A160 primary launch site

Typical Ground Control Station placement

A160 maintenance area (Hangar 5)