Lost Link/Mission Procedures:

The SPYDER UAV utilizes a BAI (L3) guidance system that uses 423 MHz. and 1770 MHz. as the frequencies for the up-link and down-link of the guidance system. The up-link provides pilot input and flight-plan updates. The down link provides air vehicle data/status such as heading, altitude, engine rpm, and air vehicle position.

If the air-vehicle is not receiving a signal from the ground station, the vehicle auto pilot will revert to a RTB (return to base) function and fly directly to the launch/recovery site.   The vehicle will rendezvous at a pre-determined GPS location at a pre-determined altitude approved by ATC. (typically 500 ft AGL and 0.5 NM stand off parallel to the runway being used). This loiter position will be determined by ATC during pre-flight safety brief held prior to each flight day.  The loiter position assigned will be clear of personnel and equipment on the ground.  If the air-vehicle arrives at the RTB location and a link is still not established, the air-vehicle will loiter at the RTB position. If a link can not be re-established at the RTB location the air vehicle will run out of fuel and return to earth.