- AAI RQ-7B Shadow 200

| Power plant One 28.3 kW (38 hp) UEL AR 741 rotary engine; two-blade fixed-pitch wooden pusher propeller. Fuel (44 litres; 11.6 US gallons; 9.6 Imp gallons) in fire-retardant, explosion-proof wing cells. Growth option for eventual heavy fuel power plant. Dimensions Wing span; 14 ft 0.0 in. Wing area; 24.0 ft2. Length overall; 11 ft 4.0 in. Height overall; 3 ft 2.0 in. Propeller diameter; 2 ft 5.0 in. Weights Weight empty; 252.0 – 257.0 lb. Max fuel weight; 73 lb (44L). Max payload; 60 lb. Max launching weight; 375 lb. Performance Max level speed at S/L; 110 kt. Max cruising speed; 60-110 AVO selectable. Loiter speed: at S/L; 60 kt. Optimum climbing speed; 70 kts fixed. Stalling speed at S/L; 46-53 kt. Max rate of climb at S/L; 500 ft/min fixed. Ceiling; 15,000 ft. T-O run; 820 ft. Operational radius; 50 km. Max datalink range; 125 Km/ 77.6 n miles. Max endurance; 5+ h. g limit; +3.6. -2.0. Descent rate; 900 to 1400 FPM. Turn rate; 20 degree roll. Approach Speed; 52 – 78 kts. Launch Automatic ground launch by hydraulic catapult, or by conventional wheeled take-off. | Recovery Automatic wheeled landing. Transportation Air vehicle can be dismantled and stored/transported in (2) 0.61 × 0.61 × 1.65 m (2 × 2 × 5.4 ft) containers; can be carried by four people. Complete system air-transportable in three C-130 aircraft. System composition: The Shadow 200 TUAV System consists of: two GCSs with GDTs, one PGCS with PGDT, an AVT shelter containing three AVs, a Maintenance Section Multifunction (MSM) shelter, a Launch/Recovery (L/R) Trailer, two Cargo HMMWV’s and two equipment trailers with a spare AV. US Army ground crew of 22. Operational status : Selected as winner of US Army Tactical UAV (TUAV) competition in December 1999; initial LRIP contract of US$41.8 million for four Shadow 200 systems; delivered from November 2000; option for further four LRIP systems exercised with award of US$19.4 million contract on 11 April 2001. Field qualification tests at Fort Huachuca, Arizona, completed in March 2001; IOT&E at Fort Hood, Texas, began at the end of April 2001 and was completed successfully in May 2002, at which time some 1,700 hours in 900 flights had been completed. A Lot 2 LRIP contract (US$22.3 million for five systems) was awarded in March 2002; Milestone C (approval for full-rate production) was achieved on 1 October 2002 and was expected to result in an order for nine systems during FY03 at a cost of approximately US$99 million. The RQ-7A was fielded in October 2002 to the US Army's 1st and 2nd Stryker Brigade Combat Teams (BCTs) at Fort Lewis, Washington, the 4th Infantry Division at Fort Hood, Texas, and for crew training to Fort Huachuca. All six Stryker BCTs are due to receive the Shadow system by May 2006. |
| --- | --- |

Shadow 200 System Baseline

Air Vehicles with

Payloads x 3

Portable Ground

Control Station & Data

Terminal

Ground Data

Terminal x 2

Remote Video

Terminal & Antenna x 4

GCS

AVT

L/T

GCS

E/T

T/Transport

MSM

M/T

Deployability / Transportability

C130 Transportable

Maintenance

Section

Personnel

4 x 33W (EW System Repairer)

3 x 52D  (Engine Mechanic)

Air Vehicle

Personnel/

Equipment

Transport

Personnel/

Equipment Transport

Equipment

Trailer

Equipment

Trailer

Maintenance

Section

Multifunctional

Ground Control

Stations x 2

System

Arresting

Net

Air Vehicle Transport

& Launcher Trailer

T/Transport

Personnel

1   x 35D   (Platoon Leader)

1   x 350U (UAV Warrant Officer)

1   x 96U   (Platoon Sergeant)

12 x 96 U  (Air Vehicle Operators)

TALS