Lost Voice Communications Procedures

The MQ-1 pilot has more reliable communication capabilities from their ground station than any airborne aircraft because they are not reliant upon the aircrafts electrical systems working.  The MQ-1 has a UHF/VHF/FM radio on board that the pilot uses for communication with ATC.  They also have UHF/VHF/FM radios in the ground station, which provide limited communication capability, and telephone land lines to both the tower and RADAR approach control.

In the event that all direct voice radio communications between the pilot and ATC are lost, the pilot will command MQ-1’s transponder to code 7600 and the pilot of the MQ-1 will contact ATC by the direct telephone landline and advise that radio communication has been lost.  Pilot will obtain landing clearance and land.

If a loss of radio communications is experienced on the ground, pilot will contact Tower ATC and wait for further instructions.  Tower ATC will not permit departure until radio communications is restored.

If communications is lost between the UAS pilot and the observer, the UAS pilot will immediately notify ATC and attempt an alternate means of communications (normally by cell phone).  Failure to regain communication between UAS pilot and the observer will require termination of UAS operations.