##### General Safety Rules and Flight profiles

## Brennan Field

Location

- MGRS approximate location 11SLT 2954 9287

- Field elevation is approximately 600 MSL.

# Rules

- Field is only already approved for Raven, Wasp, and Batmav SUAV flight-testing only.

- 30db attenuation is required for Puma and Raven.  20db attenuation is required for Wasp and Batmav.

- Un-attenuated flights are possible if SUAV frequencies in use are coordinated thru the Flight Coordinator.

- No more than 3 SUAV’s should operate simultaneously.

- No flights should exceed a distance of 0.7km from the SUAV operator/observer.

- An accurate boot up altitude of 600 feet MSL + or – 50 feet must be verified prior to launch.

- Flights not to exceed 1200 feet MSL or 600 feet AGL. Flights should not go below 700 feet MSL or 100 feet AGL unless taking off or landing.

-     NOTAM’s need to be filed with FSS prior to any flights.

- The Flight Coordinator or Lead SUAV operator is responsible for filing   NOTAM’s.

- Navigation waypoints must be entered into the GCS prior to flight.

Range     Bearing

Waypoint A 11SLT 2894 9287     600m       269

Waypoint B 11SLT 2889 9287     650m       269

Waypoint C 11SLT 2924 9235     600m       209

Waypoint D 11SLT 2921 9231     650m       209

Waypoint E 11SLT 2904 9283     500m        266

Waypoint L will be set to 11SLT 2944 9285

-     Flights will be terminated for adverse wind and weather conditions.

#### Flight Profile

- The SUAV will always be launched into the wind.

- After launch the SUAV will be flown to an altitude of 300 feet AGL.

- Once the SUAV has reached its climb altitude the SUAV operator can complete the flight performance test at their discretion.

- All flight performance tests must remain in the airspace defined by these MGRS coordinates. (Ranges and bearings are in reference to the GCS HOME coordinate of 11SLT 2954 9287).

Range      Bearing

1) 11SLT 2967 9290        132m          74

2) 11SLT 2993 9255        500m        128

3) 11SLT 2926 9257        420m        222

4) 11SLT 2927 9221        700m        200

5) 11SLT 2894 9251        700m        239

6) 11SLT 2884 9289        700m        271

7) 11SLT 2938 9295        174m        295

-  The SUAV will always be landed into the wind.

-  (With a west and calm wind condition) a downwind, base, and final will be

performed using left turns. With an east wind condition a straight in approach

will be executed. All approaches to land will be made from the southwest.

### Upper Field Waste Management

## Location

- MGRS approximate location 11SLT 3416 9813

- Field altitude is approximately 1000 feet MSL.

## Rules

- Field is approved for Raven, Swift, Pointer, Wasp, Batmav, Puma, and Dragon Eye.

- 30db attenuation is required for Raven, Pointer, Puma, Swift, and Dragon Eye.  20db attenuation is required for Wasp and Batmav.

- Un-attenuated flights are possible if SUAV frequencies in use are coordinated thru the Flight Coordinator.

- No more than 3 SUAV’s should operate simultaneously.

- No flights should exceed a distance of 0.7km from the SUAV operator/observer.

- An accurate boot up altitude of 1000 feet MSL + or – 50 feet must be verified prior to launch.

- Flights not to exceed 1600 feet MSL or 600 feet AGL.

- Flights should not go below 1100 feet MSL or 100 feet AGL unless taking off or landing.

-     NOTAM’s need to be filed with FSS prior to any flights.

- The Flight Coordinator or Lead SUAV operator is responsible for filing   NOTAM’s.

- Navigation waypoints must be entered into the GCS prior to flight.

###### South-East Navigation pattern

Range     Bearing

Waypoint A 11SLT 3415 9758      550m      179

Waypoint B 11SLT 3416 9748      650m      179

Waypoint C 11SLT 3463 9786      550m      119

Waypoint D 11SLT 3471 9780      650m      119

Waypoint E 11SLT 3457 9842      500m        54

(Waypoint L = Waypoint H)

###### West Navigation pattern

Range     Bearing

Waypoint A 11SLT 3360 9813     550m       269

Waypoint B 11SLT 3350 9813     650m       269

Waypoint C 11SLT 3388 9765     550m       209

Waypoint D 11SLT 3383 9757     650m       209

Waypoint E 11SL T 3457 9842    500m         54

Waypoint L = Waypoint H

- Flights will be terminated for adverse wind and weather conditions.

#### Flight Profile

- The SUAV will always be launched into the wind.

- After launch the SUAV will be flown to an altitude of 300 feet AGL. Once the SUAV has reached its climb altitude the SUAV operator can complete the flight performance test at their discretion.

- All flights must remain inside a 0.7km radius of the HOME coordinate. (Ranges and bearings are in reference to the GCS HOME coordinate of 11SLT 3416 9813).

-    The SUAV will always be landed into the wind.

# Lower Waste Management

#### Location

- MGRS approximate location 11SLT 3542 9728

- Field altitude is approximately 1000 feet MSL.

#### Rules

- Field is approved for Raven, Pointer Swift, Puma, Wasp, and Batmav.

- 30db attenuation is required for Raven, Pointer, Puma, and Swift.  20db attenuation is required for Wasp and Batmav.

- Un-attenuated flights are possible if SUAV frequencies in use are coordinated thru the Flight Coordinator.

- No more than 2 SUAV’s should operate simultaneously.

- No flights should exceed a distance of 0.7km from the SUAV operator/observer.

- An accurate boot up altitude of 1000 feet MSL + or – 50 feet must be verified prior to launch.

- Flights not to exceed 1600 feet MSL or 600 feet AGL.

- Flights should not go below 1100 feet MSL or 100 feet AGL unless taking off or landing.

- Navigation waypoints must be entered into the GCS prior to flight.

Range     Bearing

Waypoint A 11SLT 3494 9756     550m       299

Waypoint B 11SLT 3485 9761     650m       299

Waypoint C 11SLT 3494 9700     550m       239

Waypoint D 11SLT 3485 9696     650m       239

Waypoint E 11SLT 3491 9728     500m       269

Waypoint L = Waypoint H

- Flights will be terminated for adverse wind and weather conditions.

Flight Profile

- The SUAV will always be launched into the wind.

- After launch the SUAV will be flown to an altitude of 300 feet AGL. Once the SUAV has reached its climb altitude the SUAV operator can complete the flight performance test at their discretion.

- All flight performance tests must remain in the airspace defined by these MGRS coordinates. (Ranges and bearings are in reference to the GCS HOME coordinate of 11SLT 3542 9728).

Range      Bearing

1) 11SLT 3570 9757         400m          44

2) 11SLT 3570 9713         300m        114

3) 11SLT 3484 9689         700m        235

4) 11SLT 3474 9703         700m        249

5) 11SLT 3472 9727         700m        269

6) 11SLT 3480 9764         700m        299

7) 11SLT 3492 9777         700m        314

-    The SUAV will always be landed into the wind.

Fillmore

Location

- MGRS approximate location 11S LU 21930 06730

- Field elevation is approximately 400 MSL.

# Rules

- Field is only already approved for Raven, Wasp, and Batmav SUAV flight-testing.

- 30db attenuation is required for Puma and Raven.  20db attenuation is required for Wasp and Batmav.

- Un-attenuated flights are possible if SUAV frequencies in use are coordinated thru the Flight Coordinator.

- No more than 3 SUAV’s should operate simultaneously.

- No flights should exceed a distance of 0.7km from the SUAV operator/observer.

- An accurate boot up altitude of 600 feet MSL + or – 50 feet must be verified prior to launch.

- Flights not to exceed 1000 feet MSL or 600 feet AGL. Flights should not go below 700 feet MSL or 100 feet AGL unless taking off or landing.

-     NOTAM’s need to be filed with FSS prior to any flights.

- The Flight Coordinator or Lead SUAV operator is responsible for filing   NOTAM’s.

- Navigation waypoints must be entered into the GCS prior to flight.

WEST AREA:

Range     Bearing

Waypoint A 11SLU 21277 06794	 597m       081

Waypoint B 11SLU 21325 07064	 635m       106

Waypoint C 11SLU 21236 07063	 716m       102

Waypoint D 11SLU 21315 06526	 594m       054

Waypoint E 11SLU 21213 06532	 690m       058

Waypoint L will be set to 11SLU 21876 06760

EAST AREA:

Range     Bearing

Waypoint A 11SLU 22578 06741	 702m       255

Waypoint B 11SLU 22514 06448	 700m       280

Waypoint C 11SLU 22615 06448	 789m       277

Waypoint D 11SLU 22553 07008	 721m       234

Waypoint E 11SLU 22646 07001	 808m       237

Waypoint L will be set to 11SLU 21876 06760

SOUTH AREA:

Range     Bearing

Waypoint A 11SLU 21945 06097	 646m       339

Waypoint B 11SLU 21625 06173	 619m       009

Waypoint C 11SLU 21622 06016	 772m       004

Waypoint D 11SLU 22245 06166 	 678m       313

Waypoint E 11SLU 22245 06016	 812m       139

Waypoint L will be set to 11SLU 21876 06760

- Flights will be terminated for adverse wind and weather conditions.

#### Flight Profile

- The SUAV will always be launched into the wind.

- After launch the SUAV will be flown to an altitude of 300 feet AGL.

- Once the SUAV has reached its climb altitude the SUAV operator can complete the flight performance test at their discretion.

- All flight performance tests must remain in the airspace defined by these MGRS coordinates. (Ranges and bearings are in reference to the GCS HOME coordinate of 11SLU 21876 06760).

Range      Bearing

1) 11SLU 20520 07170	 1415m       092

2) 11SLU 21060 05550	 1453m       019

3) 11SLU 22950 05660	 1534m       301

4) 11SLU 23345 07258	 1550m       237

- The SUAV will always be landed into the wind.

- With a west and calm wind condition, a downwind, base, and final will be performed using left turns. With an east wind condition a straight in approach will be executed. All approaches to land will be made from the southwest.

Taft

Location

- MGRS approximate location 11SKU 81660 89770

- Field elevation is approximately 600 MSL.

# Rules

- Field is only already approved for Raven, Wasp, and Batmav SUAV flight-testing.

- 30db attenuation is required for Puma and Raven.  20db attenuation is required for Wasp and Batmav.

- Un-attenuated flights are possible if SUAV frequencies in use are coordinated thru the Flight Coordinator.

- No more than 3 SUAV’s should operate simultaneously.

- No flights should exceed a distance of 0.7km from the SUAV operator/observer.

- An accurate boot up altitude of 1000 feet MSL + or – 50 feet must be verified prior to launch.

- Flights not to exceed 1800 feet MSL or 700 feet AGL. Flights should not go below 700 feet MSL or 100 feet AGL unless taking off or landing.

-     NOTAM’s need to be filed with FSS prior to any flights.

- The Flight Coordinator or Lead SUAV operator is responsible for filing   NOTAM’s.

- Navigation waypoints must be entered into the GCS prior to flight.

WEST AREA:

Range     Bearing

Waypoint A 11SKU 80988 89765	 714m       075

Waypoint B 11SKU 81046 90048	 715m       099

Waypoint C 11SKU 80960 90053	 798m       097

Waypoint D 11SKU 81022 89492	 724m       054

Waypoint E 11SKU 80913 89485	 833m       056

Waypoint L will be set to 11SKU 81660 89770

EAST AREA:

Range     Bearing

Waypoint A 11SKU 82324 89750	 616m       245

Waypoint B 11SKU 82268 89415	 648m       286

Waypoint C 11SKU 82374 89405	 749m       282

Waypoint D 11SKU 82278 90056	 638m       067

Waypoint E 11SKU 82384 90051	 730m       231

Waypoint L will be set to 11SKU 81660 89770

- Flights will be terminated for adverse wind and weather conditions.

#### Flight Profile

- The SUAV will always be launched into the wind.

- After launch the SUAV will be flown to an altitude of 300 feet AGL.

- Once the SUAV has reached its climb altitude the SUAV operator can complete the flight performance test at their discretion.

- All flight performance tests must remain in the airspace defined by these MGRS coordinates. (Ranges and bearings are in reference to the GCS HOME coordinate of 11SKU 81706 89754.

Bearing/ Range

  - 11SKU 80620 90380	    104/1238m

  - 11SKU 80610 89180	    046/1240m

  - 11SKU 82960 89040	    285/1449m

  - 11SKU 83070 90210	    237/1428m