Visual Surveillance

Ground based visual observers will be located at various locations within the Cannon Class D airspace,  which may include the air traffic control tower, providing constant "eyes-on" of the UAS to ensure deconfliction from other potential nonparticipating traffic.  Observers meet all qualifications to include medical certification and prescribed 14 CFR Part 91 training on rules and responsibilities when operating near other aircraft and right-of-way rules. Observers are well versed on proper communication with the pilot and will maintain two-way communications.

Any time an observer gives instructions to the pilot to avoid potential confliction with another aircraft; the pilot will relay his actions to the air traffic controller.

If the observer loses sight of the MQ-1, the Observer will notify the MQ-1 pilot immediately, the MQ-1 pilot shall immediately relay this to ATC.  If visual observation cannot be re-established by at least one observer, the flight will be terminated.

Lastly, pilot has the ability to slew cameras to search in response to traffic advisories.