SCLA Lost Communications Procedures:   Lost Communications is highly unlikely as each operator and observer carry a hand-held radio and cell phone as a back-up means of communication with the Raven operator/mission commander.  For communications with VCV ATCT, the mission commander monitors the hand-held (FM Net) radio issued by VCV ATCT and a cell phone.  The cell phone will be Primary Comm as directed by VCV ATCT.

SCLA Lost-Link Procedures:   (There are three possible lost-link settings)

- “Go-to-Rally” - The Raven may be programmed to “go-to-rally” and auto-land.  The “rally” point will be located over the GCS within approved SCLA COA airspace and is the most commonly used procedure.

- “Land-Now” - The Raven can be programmed to auto-land within 3 seconds when the loss of link is detected by onboard programming or the Ground Control Station (GCS).

- “Finish Flight” (NAV Mode) – This option allows the aircraft to continue its navigation loop (finish at Point D, proceed to Point E and land at Point L when the flight loop time has expired).   If the aircraft is in an orbit when the LOL occurs, it will continue to orbit until the flight loop time has expired then proceed to Point E and land at Point L.

SCLA Emergency Procedures:   Specific emergency procedures are situation dependent.  In general, Raven operators have the ability to take direct (manual) control of the aircraft and return it to the GCS, or in the event of severe malfunction or erratic behavior, the operator has the option to terminate engine power and force to Raven to autoland.  All procedures will be confined to the approved COA airspace and VCV ATCT will be notified of any emergency situation ASAP.