What happens when command link is lost?

The operator can pre-program one of three options.  Figure 1 below depicts a diamond default patter with four waypoints (A – D), a Rally waypoint (E), and a Home waypoint (L).  Explained below is a summary of events for each lost link option.

Option 1 is Land Now or Autoland: The air vehicle executes the Autoland function immediately. With this option, the Raven will autoland at its present location.  The operator can/will use FALCON VIEW to determine the Ravens last know coordinate.  The hand-held controller will also display the vehicles coordinates when autoland was activated.  Additionally, the Raven will be fitted with a FALCON TRACKER – a battery operated tracker that will allow the operator to locate the aircraft.

Option 2 is Go to Rally: The air vehicle flies to the specified Rally waypoint automatically set 20 meters to the East (can be manually changed) of the Home waypoint (Home is where the GCS is located).  The Raven upon reaching the Rally waypoint will proceed to the Home waypoint, upon reaching the Home waypoint the Raven will execute an autoland.  The Rally and Home points will change as the GCS moves over the water (GCS will be on a boat during over water operations).  Regardless of where the GCS is located, the Rally point will normally be 20 meters to the East of the GCS and Home will be over the GCS.  The Rally and Home waypoints must be pre-set before launch.

Option 3 is Finish Flight: The air vehicle will continue to fly the programmed waypoints.  The Raven will continue fly to the waypoint loop until cycle time expires.  Cycle time is synonymous with loop time.  The flight loop time settings determine when the Raven will conclude its final navigation loop at last waypoint, proceed to the Rally waypoint, and then land at the Home waypoint.  The loop time is typically used as a safety backup so that the Raven is not commanded to fly beyond the life of the battery.  When the cycle time expires, the Raven will fly to the Rally point (automatically set to be 20 meters to the East of the Home waypoint).  Upon reaching the Rally point, the Raven will proceed directly to the Home waypoint (over the GCS) and autoland.

Figure 1.

Example of Diamond Default pattern with four waypoints (A – D), a Rally waypoint (E), and the Home waypoint (L).

How does vehicle respond if link is never re-established?

In all three options/modes, the end result is that the air vehicle has landed.

How does the vehicle recognize that loss of command link has occurred?

The Air Vehicle monitors the number of link packets that it receives from the GCU.  The frame rate is roughly 10 per second.  If the Air Vehicle receives zero packets per second, it would recognize that a loss of link has occurred. There are two numbers displayed on the GCU that describe the quality of the link.  The first number is the number of packets sent up by the GCU (number the aircraft received), the second number is the packets sent down from the Air Vehicle (number the GCS received).  A  9-10 status on both is good.  Anything down to about 2-3 on either up or down still results in a working up and down link.  The numbers are directly related to the link bar indication.  The bar is basically a 0 to 10 scale.  The link bar indicates the least of the two RT values.  So a 10/5 or a 5/5 would show up as a 50% link bar on the aircraft display.  A 10/5 also indicates a strong uplink, and downlink with some loss.  A 5/5 indicates some uplink loss, but still a good downlink.

How does the UAV operator in the ground control station recognize loss of command link has occurred?

The Link Status Bar on both the Aircraft and Mission Controller is a continual display of the quality of the uplink and downlink.  If the command link is lost, a warning message (clock icon) is displayed on the GCU controllers.  Air Vehicle will send a loss of link “En Route Rally” message to the operator.