What happens when command link is lost?

The Pilot must select one of three preprogrammed loss link options depending on the mission/location/situation.  The three settings for lost link are 1) finish flight, 2) return to a rally point -- normally over the GCS, and 3) land now (will autoland within 3 seconds).

For return to a rally point, here is what to expect:  If a loss of link

occurs at anytime during the mission, the Raven will fly to a predetermined rally point at a predetermined altitude (typically 300 ft AGL).  The rally point is set as the same point as the location of the GCS.  During the return flight, if link is reestablished, the pilot will make a determination on whether to continue with the mission or land the aircraft.

How does vehicle respond if link is never re-established?

If link is not reestablished, the Raven will return to the rally point (over GCS) and autonomously set itself into "autoland".  The rally point will all depend on wind and what part of Fentress we are operating.  The normal loss link rally altitude is 300 feet AGL.

How does the vehicle recognize that loss of command link has occurred?

The Air Vehicle monitors the number of link packets that it receives from the GCU.  The frame rate is roughly 10 per second.  If the Air Vehicle receives zero packets per second, it would recognize that a loss of link has occurred. There are two numbers displayed on the GCU that describe the quality of the link.  The first number is the number of packets sent up by the GCU (number the aircraft received), the second number is the packets sent down from the Air Vehicle (number the GCS received).  A  9-10 status on both is good.  Anything down to about 2-3 on either up or down still results in a working up and down link.  The numbers are directly related to the link bar indication.  The bar is basically a 0 to 10 scale.  The link bar indicates the least of the two RT values.  So a 10/5 or a 5/5 would show up as a 50% link bar on the aircraft display.  A 10/5 also indicates a strong uplink, and downlink with some loss.  A 5/5 indicates some uplink loss, but still a good downlink.

How does the UAV operator in the ground control station recognize loss of command link has occurred?

The Link Status Bar on both the Aircraft and Mission Controller is a continual display of the quality of the uplink and downlink.  If the command link is lost, a warning message (clock icon) is displayed on the GCU controllers.  Air Vehicle will send a loss of link “En Route Rally” message to the operator.