VISUAL SURVEILLANCE/DETECTION CAPABILITY

For the purpose of see-and-avoid, visual observers must be utilized at all times except in Class A airspace. The observers may either be ground based or in a chase plane. For MQ-1 operations, the unmanned aircraft (UA) must remain within a lateral distance of no more than 2 nautical miles and 3,000 feet vertically from the ground observer.

Airborne based (chase aircraft)

• Two-way communications (UHF & VHF) will be established between the UAS Pilot and the observer in the chase aircraft.

• The observer will NOT be the PIC of the chase aircraft

• Observers are trained to the same standard as the pilot with regard to “see and avoid” procedures and techniques

• To the extent consistent with the safety of the chase aircraft, the chase aircraft should be operated within one mile laterally and 3000 feet vertically from the UA.

Ground Based

• Visual Observers will utilize the 2 mile and at or below 3,000’ criteria

• The Visual Observers will have direct communications with the UAS PIC.

• The ratio of UAS to Visual Observer will be one-to-one

• The Visual observers are trained to be familiar with the RPA and to be proactive in making evasive maneuver calls.

Forward or side looking cameras

• Nose mounted forward looking camera with a 30 deg field of view.

• Raytheon Multi-spectral Targeting System EO/IR turret camera with 360 deg air to air viewing/scanning capability.