147 RW MQ-1 Lost Link Procedures at Ft Polk AAF / R-3804

BACKGROUND

The MQ-1 Predator system is designed with the ability to fly manually or autonomously.  Under normal operations, manual flight is via inputs from the pilot through use of a control stick, rudder pedals and power lever.  Much like traditional aircraft, direct manipulation of the control stick fore and aft will, in turn, direct movement of the tailboards so as to produce a climb or descent.  Likewise, movement of the control stick left or right will direct movement of the ailerons to produce a left or right banked turn.  And, of course, movement of the power lever produces changes in aircraft power.

Autonomous flight is flight which occurs via the autopilot where missions are designed and uploaded into the autopilot memory for execution.  There are two types of missions available for input: operational and emergency.

Operational missions are those which specify a geographic route of flight and can be up to 255 waypoints.  Each waypoint contains the specific information of longitude and latitude, altitude, airspeed, sensor configuration and many others.  Waypoints can be added, deleted and/or modified during flight.  The pilot elects to fly an operational mission by turning on the autopilot (doing so will override any control stick and/or power lever input).  Only operational missions that are loaded within the autopilot memory can be executed – each time a new operational mission is loaded, it replaces the previous mission (only 1 operational mission can be stored in the aircraft autopilot memory at a time).

Emergency missions can have up to 100 waypoints.  Each waypoint contains the same specific information as operational missions (longitude, latitude, airspeed, mode 3 codes, etc.). Likewise, emergency mission waypoints can also be added, deleted and/or modified during flight.  However, unlike operational missions, emergency missions are not designed to be executed through pilot election.  Emergency missions automatically execute whenever the aircraft senses the uplink signal between the aircraft and the ground control station is lost, regardless of the current mode of flying (manual or autonomous).  Therefore, emergency missions override both manual flight and operational missions.

Like operational missions, each time a new emergency mission is uploaded into aircraft memory, it replaces the previous emergency mission (only 1 emergency mission can be stored at a time).  As a safety precaution, anytime an emergency mission is non-existent within the aircraft memory (i.e. the pilot has not uploaded an emergency mission at least once), a warning is displayed.

There is no specific flight termination procedure for the MQ-1 Predator.   The design of every Predator system is such that once the aircraft passes the last point of the emergency mission the aircraft will continue to fly the last six waypoints, until the uplink is regained.  While flying the last six waypoints, if an uplink with the aircraft if unattainable and the aircraft runs out of fuel, the aircraft kills the engine and lowers the landing gear while continuing to fly the routing and airspeed of the last six waypoints as the aircraft descends into the terrain.

In the event the aircraft loses link with the ground control station either in the pattern or in the range, the pilot will maintain two way communication with ATC at all times.  Primary communication with ATC is accomplished through 2 UHF/VHF radios and antennas in the ground control station, or through one UHF/VH/FM radio onboard the aircraft.  Backup communication with Polk AAF ATC and range airspace controllers is maintained through the use of handheld UHF/VHF radios and telephones in the ground control station and at the operations center.

FORT POLK OPERATIONS

For the particular flying operations at Ft Polk, there are essentially two separate “conditions” or phases of flight which must be accommodated for with regard to emergency missions: flight in the local pattern and flight within the restricted airspace (R3804). A graphical depiction of the lost link procedures is contained at the end of this document.

Local Area/Pattern Emergency Mission Waypoints

Anytime the aircraft is in the pattern and there is a loss of uplink, the aircraft will immediately commence a climb to 1000’ MSL and fly directly to the entry (1st) waypoint of the emergency mission currently resident within the autopilot memory.  The entry waypoint of the emergency mission anytime the aircraft is located within Class D airspace (within the local pattern area) will be located 1 NM from the departure end of the active runway.  Specifically, when runway 16 is active, the entry waypoint of the emergency mission will be point LL16 (N3101.4 W09310.9) located southeast of the airfield.  Likewise, point LL34 (N3103.9 W09311.9) will be the entry waypoint for the emergency mission for runway 34 (see Emergency Mission Layout graphic).  The altitude for the entry waypoint is 1000’ MSL (700’ AGL).

As per guidance from the Ft Polk airspace managers, all local pattern traffic must remain west of the field.  Therefore, the 2nd waypoint of the emergency mission is located west of the runway - Pt 2 (N3102.4 W09312.9) assigned an altitude of 1000’ MSL.  If the uplink with the aircraft has not been restored by Pt 2, then the aircraft will exit the Class D airspace and fly east to the 3rd waypoint to facilitate troubleshooting of the data-link system, conserve fuel and move the aircraft to a safer altitude over a less populated area.  The 3rd waypoint of the emergency mission is Pt 3 (N3102.6 W09311.0) assigned alt of 2500’ MSL which is collocated with the 1st point of the normal departure procedure as described in the Ft Polk LOA.

Restricted Airspace Emergency Mission Waypoints

Anytime the aircraft is located within the restricted area (or has executed the emergency mission while in the local pattern and has reached Pt LRS) and no uplink is present between the aircraft and control station, the aircraft will proceed direct to Pt 4 (N3102.8 W09303.8) while climbing to or remaining at the pre-coordinated “working” altitude as assigned by Ft Polk Range Control (normally 8000’-9000’ MSL).  Pt 4 is the 2nd waypoint of the normal departure procedure as described in the Ft Polk LOA.

Because of the design of the system to continually fly the last six waypoints of the emergency mission after the last point has been passed, the last six waypoints of the emergency mission are arranged in a circle located within the restricted airspace (see Emergency Mission Layout graphic).  After reaching Pt 4, if the uplink with the aircraft has not been restored, the aircraft will continue on to these last six waypoints.  The specific coordinates of the remaining emergency mission waypoints are:

Pt 5 (N3105.9 W09302.5)

Pt 6 (N3106.7 W09302.5)

Pt 7 (N3106.7 W09300.8)

Pt 8 (N3105.9 W09300.0)

Pt 9 (N3105.1 W09300.3)

Pt 10 (N3105.1 W09301.6)

Altitudes for points 5-10 are as assigned by range controllers.

Once the aircraft reaches Pt 10, it will fly to Pt 5 and continue the flying the last 6 waypoints.

NOTE: The last six waypoints of the emergency mission are located within the Redleg Artillery Impact Area.  This area is free of civilian personal property (houses, etc.) and requires coordination with Ft Polk Range Control prior to personnel entry; thereby minimizing the chances of injury and/or personal property damage in case of a failure to regain uplink with the aircraft (and fuel starvation). While flying the last six waypoints, if an uplink with the aircraft if unattainable and the aircraft runs out of fuel, the aircraft kills the engine and lowers the landing gear while continuing to fly the routing and airspeed of the last six waypoints as the aircraft descends into the terrain.

CONCLUSION

This arrangement of waypoints and altitudes will ensure that regardless of where the aircraft is located during flight, if the uplink with the aircraft is lost, it will immediately execute an emergency mission that will ensure the aircraft begins safely climbing away from the ground and/or maintains a safe altitude, direct it to fly to specific coordinates that will help facilitate re-acquisition of an uplink signal, return the aircraft to restricted airspace (if in Class D airspace at the time of lost uplink) and ensure that it remains there.  If an uplink is not regained, the aircraft will “land” in an artillery impact area which requires coordination with Ft Polk Range Control prior to entry of personnel and is free of civilian personal property.