Emergency Procedures

USAF Unmanned Aircraft Systems (UAS) Pilot In Command (PIC) emergency procedure actions are extensively covered within USAF Technical Order 1Q-1/9 (M)B-1, AFI 11-2MQ-1/9 Volume 3, AFI 11-214 and Cannon AFB Local Flying Procedures.  The UAS PIC is required to operate in accordance with these regulations while coordinating appropriately with ATC and abide by CFR 91.3 (responsibility and authority of the pilot in command).  USAF will notify the appropriate Air Traffic Control (ATC) facilities of any changes to these documents to ensure continued safe operations.  Cannon ATC facilities will maintain specific emergency procedures in the facilities’ ready reference files and watch supervisor's emergency procedures checklist.  The following are excerpts from USAF governing regulations listed above:

Forced Landing:

- When landing off the airfield, prioritize actions to minimize collateral damage, injury, and/or loss of life.

- Forced Landing.  Crews will comply with forced landing procedures in the flight manual and local guidance.  If a forced landing to a runway or primary site cannot be completed without endangering personnel or property on the ground, then another site will be selected even if attempting a landing at that alternate site may result in destruction of the MQ-1 aircraft.

Air Aborts:

- Abort the mission, regardless of apparent damage or subsequent normal operation for any of the following: bird strike, over-G, uncommanded flight control inputs, or engine failure.

In-Flight Practice of Emergency Procedures:

- Pilots will practice aborted takeoffs and unusual attitude procedures only in the flight simulator or cockpit procedures trainer.

- Practice in-flight engine shutdown is prohibited.

- Initiate go-arounds from Simulated Flameout (SFO) patterns in sufficient time to ensure the aircraft does not touch down.

In addition to these USAF regulation directives, UAS PIC’s operating within the National Airspace System (NAS) outside of restricted airspace will utilize the following procedures to mitigate risk in the event of an UAS In-Flight Emergency (IFE):

Mitigation Procedures for ALL Operations: (regardless of IFE)

- UAS must be shown to be airworthy to conduct flight operations in the NAS.

- UAS shall operate with an operational mode 3/A transponder, with altitude encoding, set to an ATC assigned squawk.

- UAS shall operate with position/navigation lights on at all times during flight.

- Operations, including lost link, will not be conducted over populated areas, heavily trafficked roads, or an open-air assembly of people.

General IFE Mitigation Procedures:

- During IFEs the UAS Crew will utilize all available UAS sensors (I.E. all camera’s) to visually scan for potential conflicting traffic.

- PIC will immediately report all IFEs to the appropriate ATC facility and squawk 7700 (7601 for Lost Link).  Upon request the PIC will provide ATC with the standard minimum emergency information to include: aircraft position and altitude, nature of emergency, pilot intentions/requests, and fuel remaining in hours.

- PIC will request IFR ATC service if aircraft will descend through FL 180.

- The PIC will assess the ability to safely recover the UAS to Cannon AFB.  If a safe recovery

is unlikely the PIC will choose an appropriate Forced Landing Location.  Melrose is the primary desired locations for UAS Forced Landings.

Emergency Procedures within Cannon AFB Radar Approach Control Airspace:

- A dedicated controller will control and monitor the unmanned aircraft while scanning the projected flight path for primary and secondary targets that may be in conflict.  The controller will cooridinate appropriate actions for participating traffic and will attempt to advise any non-participating traffic on both ATC and Guard frequencies.  ATC may also vector the MQ-1 to mitigate the conflict.

- When other unmanned aircraft are airborne in the pattern, ATC will assure deconfliction.  Established holding patterns exist within the Class D airspace and are well clear of populated areas.  ATC will direct the UAs’ to use these holding patterns for either the emergency UAS whether they are “troubleshooting” or awaiting clearance to land.  These holding points may also be used if required to clear additional UAS out of the pattern traffic.

- If a Cannon AFB assigned manned aircraft is within the vicinity of the emergency MQ-1and if the MQ-1 PIC concurs, the MQ-1 crew will coordinate with ATC and the Cannon Command Post to obtain approval for the manned aircraft to provide flight escort.  This is an added risk mitigation enabling an increased see-and-avoid capability.  During escort operations the PIC of the manned aircraft will operate in accordance with USAF guidance for separation from the UA.  When escorting, the PICs of both aircraft must remain in direct radio communications.

- Cannon AFB Crash Net and response procedures are unchanged for UAS emergencies.