Emergency Procedures MQ-9

In case of lost link, the pilot shall:

Inform ATC that the aircraft is lost link and state intentions.

Aircraft will fly the preprogrammed mission, to include a 15 minute orbit at the lost link point, or within the restricted airspace it will enter after departing the Valmont ATCAA.

If link cannot be reestablished, aircraft will squawk 7700, and fly the pre-programmed route, usually reverse routing unless otherwise specified, to Holloman AFB.

Aircraft will fly back to Holloman AFB, where it enters the “last six waypoints”, and is recovered via line of sight C-band command and control link.

Should control not be established, aircraft will stay in orbit until fuel is depleted and it will then impact the ground in the orbit area, inside restricted airspace.

If MQ-9 engine failure occurs in R-5107 A/B/D, it will assume maximum range glide speed, remain in restricted airspace and attempt RTB to Holloman. Pilot will follow T.O. procedures for engine restart. If engine fails aircraft will be crash landed within restricted airspace.

If MQ-9 engine failure occurs while within Valmont ATCAA, aircraft will assume maximum range glide speed, proceed on heading until within restricted airspace and pilot will follow T.O. procedures for engine start.  Aircraft will not descend below FL 180 while within confines of Valmont ATCAA.  If engine cannot be restarted, aircraft will be crash landed within restricted airspace.

If MQ-9 engine failure occurs in R-5103 B/C, it will assume maximum range glide speed, and if altitude does not permit crossing Valmont ATCAA to exit on the west above FL 180, it will remain in R-5103 B/C and will be crash landed within that restricted airspace.

At no time will the MQ-9 be where it could not glide, engine out, into restricted airspace, prior to descending through FL 180.