Airworthiness Statement

Insight A-20

The Insight A-20 does not have sense and avoid capability.  Midair collision is a risk.  All flight operations shall be conducted to ensure that manned and unmanned aircraft shall not occupy the same airspace.  Airspace de-confliction shall be accomplished using local land based radar information, when available, as well as by employment of ground based observers.  CAL FIRE will have a full time airspace coordinator from the US Forest Service involved with all Insight operations.

Since the Insight A-20 system has not undergone the complete airworthiness certification process, all flights will be conducted in a manner targeted to minimize exposure to manned aircraft, populated ground areas and at least 5 miles from any airport or B, C and D airspace.

The Insight A-20 will be flown from a location within Class E or G airspace, not within 5 miles of any airport, and deployed to an incident managed by the CAL FIRE.  CAL FIRE will ensure that a TFR has been established and that the aircraft is equipped with a mode-C transponder.  Safe operations of flight will be overseen by a certified CAL FIRE Air Tactical Group Supervisor (ATGS).  The airspace will be cleared of all other aircraft within visual reference by ground personnel and managed by a CAL FIRE ATGS.

The Insight A-20 will be flown with the same attention to detail as our other Call When Needed (CWN) aircraft.  CWN aircraft are carefully and thoroughly inspected by CAL FIRE Aviation’s engineer who is an FAA qualified structural Designated Engineering Representative.  In addition to issuing an interagency data card, CAL FIRE Aviation’s Chief Instructor Pilot inspects and approves pilot operator certificates, including medical for all operators of the Insight A-20.

The following CAL FIRE statement details the procedures used to ensure safe operation of the Insight A-20 vehicle related to fires in California and follows the CWN procedures established to qualify pilots and equipment used in the defense of wildland fires.

| Scope: This memorandum constitutes an interim airworthiness statement, authorizing operation of the Insight Unmanned Aerial System (UAS) to be deployed in support of wildand fire management in California under the supervision and control of the California Department of Forestry and Fire Protection (CAL FIRE). This airworthiness statement is relevant to all FAA Certificate of authorization (COA) that may be released for Wildland firefighting for Temporary Flight Restriction (TFR) covered by 14 Code of Federal Regulations (CFR) 91.137 (a) (1) File Code: | 8300 |  | Date: | June 27, 2008 |
| --- | --- | --- | --- | --- |
| Route To: |  |  |  |  |
| Subject: | Interim Airworthiness for Insight Unmanned Aerial System (UAS) using the |  |  |  |
|  | Insight A-20 Unmanned Aerial Vehicle (UAV) |  |  |  |
| To: | Federal Aviation Administration (FAA) On-line COA review |  |  |  |

Validity: This airworthiness statement can be terminated upon issuance of a later airworthiness statement or sooner if necessary by CAL FIRE..

List of Appendices:

Appendix A – Restrictions, Operating Information and Control Segment

Appendix B – Configuration Details

Appendix C – Inspection, Maintenance, and Logbook Instructions

Appendix D – Risk assessment

The point of contact for this statement is Dave Wardall (Air Operations Officer III, Deputy Chief Aerospace Engineering and Manufacturing) 916-561-3331, office, 916-531-1770 mobile, 916-762-2802 pager, and 916-561-3354 fax.  Email address is Dave.Wardall@fire.ca.gov

Original signature on file

/s/ W. David Wardall

W. David Wardall

Deputy Chief Aerospace Engineering and Manufacturing

Appendix A Restrictions

WARNING

The air vehicle is susceptible to radiated emissions. Loss of link, loss of GPS, loss of video, and possible loss of vehicle are possible in proximity to emitters.

WARNING

Accidental operation of the UAS outside of the Fire TFR and the FAA approved COA shall be immediately reported to Air Traffic Control (ATC) and the appropriate local on scene aircraft flight following personnel.

WARNING

The UAS does not have sense and avoid system. Mid-air collision is a risk. All flight operations shall be conducted to ensue that manned and unmanned aircraft shall not occupy the same airspace.

WARNING

The UAS has not completed full airworthiness qualification. All flight operations shall be conducted in a manner to minimize exposure to manned aircraft and ground resources.

WARNING

When UAS operations rely on ground observers or aerial observers to provide airspace de-confliction, radio contact between the ground observer or aerial observer and the aircraft operator must be maintained. If radio contract between ground observer and aircraft operator is lost, the AV shall return to the home waypoint and loiter until communications are re-established.

- Operate the aircraft system following the systems operator’s manual.

- The rally waypoint shall be inside the approved COA airspace and over an approved auto land point.

- In case of loss of communications between the aircraft operator and the local Air Traffic Control (ATC) authority, the aircraft shall be directed to the rally waypoint or as directed in the approved COA.

- Verified loss of any aircraft or Ground Control Unit (GCU) flight critical sub-system shall require return to base (RTB).

Appendix A Restrictions (Page 2)

- Any unexplained anomaly shall initiate RTB. No in-flight trouble-shooting is authorized.

- Local ATC shall be briefed before each flight with expected mission to aid in de-confliction or as described in approved COA.

- Flight over populated areas should be minimized but is authorized for fire emergency.

- During preflight, someone other than the operator shall verify that the rally waypoint is entered correctly.

- Emergency procedures: In accordance with the operator’s manual.

Appendix B – Configuration Details

- Configuration details: The UAS aircraft systems are defined in the operators manual.

- Special installation instructions: See Operator’s Manual for payload and configuration installation.

- Installation verification and quality checks: See Operator’s Manual for installed equipment.

Appendix C – Inspection, Maintenance, and Logbook Instructions

- Conduct inspections and maintenance as specified in the operator’s manual.

- The pilot shall complete log book entries after completion of daily flights for the UAS.

- CAL FIRE will complete a/c and system inspections with the following check list to verify safety of system and program. All aircraft and operators will be “carded” by an authorized CAL FIRE manager.

CAL FIRE UAV INSPECTION PROCEDURES

Aerospace Manufacturing and Engineering Department

BACKGROUND- This inspection checklist is assembled to insure that UAV operations can be performed with assurances to safety to ground and air fireline personnel, and to insure that that UAV has system safeguards to control the aircraft in the event of multiple system failures.

UAV INSPECTION PROCEDURES- The following UAV Inspection Checklist will be used to inspect the aircraft construction, systems functionality and normal and emergency operating procedures.  This checklist excludes pilot certification and pilot operational and observer training and certification.

Section 1.0- UAV Design and Construction.

1.1	Airframe design and construction.

1.2	Airframe construction- acceptable materials and fabrication process  _______

1.3	Aircraft hardware- grade and secondary locking means acceptable  _______

1.4	Engine installation sound construction  __________

1.5	Electrical generation system installed  ________

1.6	Aircraft wiring and connector locking acceptable ________

1.7	Propeller construction and installation security _________

Section 2.0- UAV Maintenance and Inspection Programs

2.1	UAV has a prescribed Maintenance and Inspection program  _________

2.2	UAV is inspected and maintained in accordance with the above programs ______

2.3	UAV manufacturer has a Service Bulleting System for UAV __________

2.4	UAV is maintained and modified IAW factory Service Bulletins _______

2.5	Manufacturers list of Service Bulletins is current  __________

2.6	UAV has an established logbook system to record maintenance performed _______

2.7	Logbook entries are current for each UAV  __________

Section 3.0- UAV Flight Control Systems

3.1	UAV has sufficient flight control system programs control aircraft operation _____

3.2	UAV ground control vehicle has multiple sources of ground power ___________

3.3	UAV has internal program to confine operation to a pre-programmed area in the event of lost communications and control  __________

3.4	UAV has flight termination program to shut down engine if pilot-in-command loses control of aircraft  _______

3.5	Mode A and mode C transponder installed yes ____ no _____

3.6	Mode C transponder and altitude reporting operational and accurate with functional test yes ___ no ____

3.7	Emergency engine shut down demonstrated with engine operating and then shut down (flight termination functional check) yes ____ no ___

Section 4.0- UAV Operational History

4.1	UAV has an established operational history of at least 1,000 operational hours ________

4.2	UAV has a record of system modifications to maintain operational reliability and airworthiness (or condition for safe flight) _________

4.3	UAV has been used by US Military   Yes ______  No _____

4.4	UAV has been used by other Public Agencies  Yes ____  No ____

Section 5.0- UAV Aircraft and systems Certification.

This certification is specifically limited to the inspection and acceptance of the airframe, powerplant, propeller and operating normal and emergency control systems, and the maintenance and inspection programs for the UAV.  Pilot operation and certification will be provided by CAL FIRE Flight Operations.

I have inspected the UAV aircraft listed below and find that these aircraft are in a condition for safe flight for the mission to be performed and that there are adequate normal and emergency operating systems to safeguard the public and air traffic for use and operation.

_________________________________				__________

W. David Wardall, Deputy Chief					Date

Aerospace Manufacturing and Engineering

Aircraft Make:  ______________  Aircraft Model:  ____________________

Serial Numbers: 	1) _______________________

2) _______________________

3) _______________________

4) _______________________

See CAL FIRE Aircraft Data Card for Approved Operator and Operating Limitations.

Appendix D – Risk assessment

CAL FIRE does not currently have a UAS qualification program. However we have reviewed other agencies and the several DoD processes including the US Army AR 70-62. After review of SOP AE70-62-1-UAS, the UAS for interested under this memorandum fall into Level 3.

UAS QUALIFICATION LEVELS

- a. All Unmanned Aircraft Systems (UAS) and Remotely Operated Aircraft (ROA) shall be evaluated for airworthiness in accordance with the specific level of the vehicle shown in Figure 1. Aircraft Qualification Levels are generally determined by vehicle weight and maximum velocity but the level may be adjusted based on multiple factors.

Figure 1: UAS Minimum Qualification Level Chart

| Vehicle Type | Existing Regulatory Guidance (Reference only) | General Guidance |  |  | Int’l Airspace | National Airspace | DoD Airspace |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Max Weight (lbs) | Max Speed (kts) | Sovereign | FAA Class A, B, C, D, E, G | Restricted Areas & Combat Zones | Expendable UA on Range and/or Airfield (per RCC 323-99) |
| Med/Large Fixed Wing | Part 23, 25 | >1320 | >200 | 1 | 1 | 2 | 3 |
| Med/Large Rotary Wing | Part 27, 29 | >1320 | N/A | 1 | 1 | 2 | 3 |
| Light, F/W and R/W | Sport Aircraft / Ultralight | Up to 1320 | 200 | COA Process ( Addressed on case-by-case basis) |  | 2 | 3 |
| Small / Mini / Micro, F/W and R/W | Association Of Model Aeronautics (AMA) | Up to 55 | 120 |  |  | 3 | 3 |

Level 1 certifies to standards equivalent to manned systems tailored for UASs

Level 2 authorizes to standards less stringent than those for manned systems

Level 3 authorizes to a minimum acceptable level of safety

The manufacturers Risk Assessment worksheet will be used prior to each flight, additional risks will be identified and mitigated on scene.