Lost Communications Procedures

The primary radio communication between Insight A-20 and CAL FIRE will be to the Air Tactical Group Supervisor (ATGS) flying in close proximity to the Insight A-20.  Communications will be accomplished using frequencies established by CAL FIRE for tactical operations in the TFR.

In the event of lost radio communications on the primary frequencies, CAL FIRE will use their alternative AM frequencies.  If this back-up communication fails for a period of greater than 10 minutes, Insight A-20 will go to a predetermined hold point, orbiting the ground control station, where it can be visually observed by the Insight A-20 operator.  If communications with the ATGS cannot be reestablished in a reasonable time, the mission will be aborted and the aircraft recovered.