EMERGENCY PROCEDURES

As with any system emergencies are a concern. The A-20 has built in lost link timers with associated flight plans, engine out flight plans, lost GPS procedures, and many others. All emergencies have associated check lists both electronically and in paper formats. All operators will be familiar with the timers and flight plans prior to flying any mission. As with any aircraft, it is the operator/pilots responsibility to ensure that these procedures are followed in accordance with the respective check list.

In addition to these procedures, CAL FIRE will deploy an Air Tactical Group Supervisor (ATGS) in an aircraft in the TFR with both visual contact and collision avoidance transponder based identification of the A-20’s location at all times.  The ATGS will also ensure that no other aircraft are operating in the area when the A-20 is deployed.

The ATGS has complete control of the airspace and is in communications with both local ground forces and the incident command center.  This allows the ATGS to inform all aviation and ground assets of any emergencies that occur during the operation of the UAV in the COA.

| Major Emergency Types & Procedures |  |
| --- | --- |
| Emergency | Procedure |
| Launch failures | The UAV is launched in a direction and manner such that, in the unlikely event of a failed launch for any reason, the air vehicle will perform an emergency recovery. |
| Engine failure | In the unlikely event of an in-flight engine failure, the UAV is immediately directed to a pre-determined flight plan over a pre-determined safe landing zone. The UAV autonomously flies back and forth over this emergency landing zone until it makes a belly-landing. |
| Lost communications / Data link | If communications are lost with the UAV, the vehicle proceeds via a pre-determined flight path to its designated emergency hold point, where it maintains position for a pre-designated period of time. This hold point is located in the immediate vicinity of the Ground Control Station (GCS), so that while the UAV is holding, additional attempts can be made to re-establish communications with the vehicle. If communications are not re-established, the UAV determines the real-time winds, determines the best emergency landing direction to use, and then, after all timers have expired, it executes a belly-landing on the emergency landing zone. |
| Recovery | The recovery flight path direction is predicated upon prevailing winds. The SkyHook is located where numerous options are available. Should a missed approach occur, the UAV returns to the hold point and attempts another approach to recovery when commanded to do so. |
| Emergency hold | An emergency holding location has been established and pre-programmed into the UAV. If any emergency occurs, the UAV is sent to this holding pattern. Should a lost communications condition occur between the UAV and GCS, the UAV intercepts a pre-planned route to the emergency hold pattern where an attempt is made to regain control of the communications / data link. |
| Emergency landing | Should an autonomous SkyHook recovery not be possible, an emergency landing zone will have already been established to recover the UAV – one that poses minimal hazard to ground personnel and equipment. |
| Ground observer | A Ground Observer (GO) is present for all flight operations, and has direct two-way radio communications with the Operator / GCS. Both the Operator and the GO hold Class II FAA Medical Certificates and are trained/tested in applicable items of FAR part 91. The GO continuously monitors the UAV and scans the surrounding air space for other aircraft, providing see-and-avoid capability for all UAV operations within the desired area. |
| Air Tactical Group Supervisor | The Air Tactical Group Supervisor (ATGS) is trained in both aviation tactics and ground tactics in defense of wild land fires. CAL FIRE has a complete curriculum and training program to certify individuals to perform ATGS functions. This individual will be stationed in an OV-10 observation aircraft and is responsible for tasking, controlling and observing the UAV. In addition, the ATGS is in charge of all aircraft assigned to the fire. In an emergency, the ATGS will be responsible for visually and electronically tracking the aircraft via Traffic and Collision Alert Device (TCAD) back to the home site or to its emergency landing position. |
| UAV health and status | The UAV transmits the health and status of the air vehicle and its systems to the GCS, and all systems are monitored real-time by the UAV operator. Should the air vehicle parameters become degraded at any time, the operator will be notified by both visual and audible alarms, and will take the appropriate immediate action(s), initiating emergency procedures as required. |