Lost Link/Mission Procedures

The UAV has a series of mission parameters which are physically loaded into its memory prior to flight. These parameters define the locations of emergency runways, lost link flight plans, and timing and safety limits used by the aircraft in the event of lost link or lost-navigation situations. Although configurable in flight, these parameters are typically designated for a given launch and recovery site and are configured for the anticipated flight environment and mission(s).

The Insight A20 UAS is pre-programmed to maintain both lateral and vertical space within the authorized area in the event of a lost link with the ground control station.  During a lost uplink/mission scenario, the aircraft has a predetermined flight plan that it will follow with both lateral and vertical positions defined.  The aircraft will loiter in that flight plan until a predetermined timer expires, at which point the aircraft will execute an approach on a predetermined runway.  In the remote event of an engine failure (to date this has not occurred with the EUS), there is a pre-staged flight plan to assist the operator in responding to the emergency so that the operator can recover the aircraft in a suitable and safe location. Furthermore, the aircraft will execute a straight and level landing posture to ensure the lowest impact on emergency landing.

In addition to these procedures, CAL FIRE will deploy an Air Tactical Group Supervisor (ATGS) in an aircraft in the TFR with both visual contact and collision avoidance transponder based identification of the A-20’s location at all times.  The ATGS will also ensure that no other aircraft are operating in the area when the A-20 is deployed.

Insight A-20 Lost-Link Procedure

- The lost-uplink procedure begins after 10 seconds time has passed without the Insight A-20 receiving any messages from the ground. The Insight A-20 then flies 55 knots airspeed, holds its current altitude, and starts a periodic reset of its communications channels.

- The Insight A-20 continues tracking its current flightplan for 30 seconds.

- The Insight A-20 climbs for a period of time towards the highest of three altitudes: its current altitude to 1000 AGL or the altitude calculated for line-of-sight communications with the ground control station.

- The Insight A-20 continues tracking its current-flight plan for 2 minutes before returning to the ground control station.

- The Insight A-20 flies directly to the nearest waypoint in the abort flightplan (the waypoint must also be closer to ground control station). The Insight A-20 climbs to the new flightplan altitude if it is higher than the current altitude.

- After reaching the home holding pattern, the Insight A-20 waits for 20 minutes time to allow the ground crew time to re-establish communications and secure the landing area.

- If communications have still not been re-established, the Insight A-20 selects an appropriate predetermined safe approach and touchdown point for a belly-landing. This ditching selection considers the current winds as measured by the Insight A-20, and the location previously programmed into memory.