| College of Agriculture and Life Sciences Elson Shields Department of Entomology Cornell University 4144 Comstock Hall Ithaca, NY 14853-2601 | Telephone: 607 255-8428 Fax: 607 255-0939 Email: es28@cornell.edu |
| --- | --- |

Public Aircraft Statement:

Airworthiness of the current UAV design

Pedigree of the design

The design of the UAVs in this COA is based on the popular Senior Telemaster ARF used by some projects.  Previous testing in the mid-1990s indicated that the Sr. Telemaster ARF was inadequate for the defined task.  The weak points of the ARF were durability of the materials and construction in the ARF, insufficient cargo capacity, insufficient fuel capacity and undesirable wing attachment (with rubber bands).  However, the wing airfoil (modified Clark Y) and the lifting horizontal stabilizer were retained from the Sr. Telemaster design.  In addition, the alignment geometry was retained.  The horizontal stabilizer is aligned with a negative 1.5 degrees incidence relative to the wing.  This alignment contributes to the stability of the design by lifting the nose of the aircraft.  The presence of the lifting airfoil in the horizontal stabilizer allowed for the movement of the aircraft CG to be moved back to 38% of wing chord, allow for more convenient cargo loading and retain the stability qualities of the high-wing “trainer” design.

Testing and flight history of the design

The UAVs for use in this COA request were designed specifically for the type of sampling proposed in this COA request.  The aircraft design was first flown in the spring of 2001 and since the initial flight the airframe has flown more than 500 hrs without any airframe problems or failures.

Airworthiness certification of new aircraft

All airframes are built in-house by skilled builders.  Airframe inspection occurs throughout the building process and a complete inspection of the airframe is conducted before the airframe is covered with film covering material by the project leader (E. J. Shields).  All electronic components are installed by a single trained individual to insure uniform installations across all flying UAVs.

Prior to the maiden flight, all electronic systems onboard the new aircraft are checked for function and resistance to RF interference.  A flight transmitter range check with antenna collapsed with the engine running at full throttle is conducted to make sure that no RF interference is present before the first flight. All new airframes are flown on their maiden flight by the in-house test pilot (Project Leader – E. J. Shields).  Each new airframe undergoes a series of two to five test flights by the in-house test pilot until all adjustments are completed.  After the test flights are completed satisfactorily, the airframe is cleared for research flights.

Airworthiness certification of currently flying aircraft

A log book is kept for each of the 12 currently flying UAVs.  Entries are made regarding hours flown, problems which need to be resolved and any other issues necessary.  Each winter, every airframe receives a complete structural inspection, complete testing of the on-board electronics, batteries are cycled and replaced if under capacity and engine maintenance is performed. Each spring, each UAV is given at least one shakedown test flight before being cleared for research flying by the in-house test pilot (Project Leader – E. J. Shields).

Airframes are retired and replaced on a regular basis.  Decisions to retire/replace airframes are made on the basis of the annual winter structural inspection and the total number of hours flown by the airframe.  The majority of airframes are retired and replaced before 100 hrs of airtime are accumulated.  Even though the airframe is usually still structurally sound after 100 hrs of flight, the airframe is usually replaced for maximum safety.

Airframes are constructed in-house and are jig-built.  Inspection occurs throughout the construction process.  This airframe is being constructed as a replacement for the 2008 or 2009 flight season.  A wing is being constructed in the center of the picture and there are 6 additional airframes under construction in the upper left hand corner.