Attachment 5 Launch/Recovery

Launch
Upon completion of preflight checklist the Wasp vehicle is ready for launch.  The following procedures are followed to conduct a launch:

- Determine wind direction.  Launch into the wind.

- The MO will point the vehicle nose in the desired takeoff direction.

- The VO will select MAN (manual) mode.  The VO will announce “clear prop”.

- The MO will grasp the vehicle by the paylod placing fingers in the recesses provided.  See figure 5.   The MO will acknowledge with “prop clear”.

Figure 5.  Wasp Launch Grip

- The VO commands 100% throttle.

- The MO will wait approximately three seconds for motor to reach full rpm.  After motor reaches full rpm, MO will then throw the vehicle into the wind.  The MO will throw it upward at an angle between 10 and 25 degrees.  See figure 6.

Figure 6.  Wasp Launch Diagram

- The VO must be prepared to provide joystick input to adjust for desired flight path.  However, if the launch appears to be unrecoverable the VO must be prepared to command auto land immediately.

- The VO maintains the climb out into the wind until clear of obstacles.  At that point the VO will select the appropriate flight mode for the mission being undertaken.

Recovery

The Wasp Air Vehicle may be manually or automatically landed. The Air Vehicle should always be landed into the wind to help slow forward motion and reduce impact of touchdown.

To perform a manual landing:

1. Make final approach in ALT mode. Recommended altitude for

final approach is 50 ft. AGL.

2. Command Autoland.  Motor stops and Wasp glides to the ground.

To perform an autonomous landing:

1. Place Air Vehicle in NAV mode.

2. Reroute Air Vehicle to Waypoint E.  Wasp will use GPS to fly to waypoint E then descent to land at waypoint L

It is possible to safely abort landing at any time.