Lost communications procedures

The pilot in command (PIC) and observer(s) will be equipped with 800 MHz trunked public safety radios for direct communication.  The radios are equipped with access to a conventional non-trunked repeater system in a separate physical location.  The pilot and observers will also be equipped with departmental issued cell phones.  The PIC will have an aircraft band radio tuned to the local ATC frequency and a second aircraft band radio tuned to the guard frequency.

In the event of loss of the trunked radio system UAS crew members will immediately switch to the backup non-trunked system.  The mission will be allowed to continue.

In the event of a radio failure (public radio direct communications system) the PIC and primary observer will immediately initiate a cell phone call.  The mission will be terminated and the UAS commanded to return to the recovery point and land.

In the event of a failure of the aircraft band radio (loss of communications with ATC) the PIC will announce termination of the mission and command the UAS to return to the recovery point and land.  The PIC will utilize a department issued cell phone to make contact with ATC as required.