Launch/recovery

The Rotomotion SR30 UAS is capable of automatic takeoff and landing.  The aircraft can be landed manually if necessary.

Takeoff procedure:

Follow checklist to preflight aircraft, setup computer and communications

Check UAS systems for proper operation

Safety pilot checks operations area and calls for everyone to stand clear

Safety pilot activates a switch on the 72MHz safety transmitter: This enables the aircraft

Safety pilot hands off control to the CGS pilot (Pilot in command)

The PIC commands the aircraft to takeoff

The aircraft automatically climbs to 15 feet and awaits command from the PIC to execute the mission

Upon command the aircraft climbs to mission altitude either straight up or climb and depart to the operations area depending upon programmed mission profile

Recovery procedure

After mission completion the aircraft will recover to the landing point

The PIC commands the aircraft to descend to 15 feet over the landing point

After verifying conditions are safe to land, the PIC pushes a button to put the aircraft in landing mode

The PIC commands final descent by holding the final descent button

Upon the aircraft contacting the ground the PIC ends the landing sequence

The aircraft propulsion system shuts down

The PIC passes control to the safety pilot

The safety pilot deactivates a switch on the 72MHz safety transmitter to disable the aircraft

The safety pilot approaches the aircraft, disarms it and announces mission completion