Lost link procedures:

The GCS creates and sends a “watchdog” data packet to the UAS every second.  Via the GCS the pilot in command (PIC) sets the link failsafe parameters (failsafe recovery location, altitude, and transit speed) and the time to failsafe in seconds.  The failsafe time out is user selectable between 10 and 30 seconds.  The North Little Rock Police Department has selected 10 seconds as the fail safe timeout.  The maximum speed of the UAS is 35 knots.  The department has selected 10 knots at the failsafe transit speed.  When the link is down for the specified period of time, the failsafe behavior triggers and returns the UAS to the failsafe recovery location programmed for the mission.  In practice, this point is close to the area where the UAS operations are taking place.  Generally once the UAS is returning to the GCS location via the failsafe system the link will reestablish.  If the link is not reestablished, the safety pilot can preempt the flight control system and the GCS operator (PIC) by means of a switch on the 72MHz safety transmitter.  The safety pilot assumes the role of observer and PIC at this point.  If the UAS is in visual range of the GCS operator, the GCS operator assumes the role of additional observer. The safety pilot then has manual control of the aircraft and can land it manually, either at the planned recovery point or any suitable location near the flight path.  The UAS is not permitted to fly beyond the range of the safety pilot (either the range of the safety controller, or the range the safety pilot can effectively control the UAS).

As part of the mission planning procedures alternate emergency manual control landing locations will be identified along the planned flight path.

Lost link parameters:

Failsafe timeout: 10 seconds

Failsafe recovery location: To be determined as part of mission planning.

Failsafe altitude:  300’ AGL

Failsafe transit speed:  10 knot

Recovery point hover altitude:  15’

Emergency manual landing sites:  To be determined as part of mission planning.

Fly away potential:

The UAS has a low potential for uncontrolled fly away.  During mission planning the flight profile is loaded into the UAS flight control system.  After loading the flight profile the GCS is only necessary for control of the UAS in the event the UAS flight path needs to be changed during the mission and during landing operations.  The flight profile includes a preplanned recovery point in the event of lost link.  If lost link occurs the UAS autonomously returns to the recovery point while seeking to reestablish the link.  If the link is not reestablished the UAS enters hover upon reaching the recovery point, descends to the recovery point hover altitude and awaits manual control to a landing.  In the event of a flight control system failure, the UAS is equipped with a manual control system utilizing a standard radio control system operating on an approved 72 MHz model aircraft control frequency.  In the event of a flight control system failure the safety pilot becomes the UAS PIC and manually controls the UAS to a landing at a suitable location.

The Rotomotion SR30 UAS is a rotary wing UAS.  Unlike fixed wing aircraft which are typically stable, the SR30 rotary wing UAS is inherently unstable and requires constant minor adjustment to maintain flight.  In the event of a total flight control failure,  (total loss of the onboard automatic flight control system, total loss of flight control system backups, and total loss of the backup manual safety control system) the aircraft becomes unstable and will end flight typically 20 to 30 yards from where total flight control failure occurs.