Emergency procedures

Lost Link

Terminate the mission. Failsafe triggers after timeout and the UAS returns to the programmed failsafe recovery point.  If link is regained, PIC commands auto land.  If link is not regained the safety pilot takes manual control and lands the UAS.

GCS computer hardware or software failure

Terminate the mission. Failsafe triggers after timeout and the UAS returns to the programmed failsafe recovery point.  The safety pilot takes manual control and lands the UAS.

Video/Camera failure

Terminate the mission.  The pilot in command (PIC) commands the UAS to return to the recovery point and auto land.

Automatic flight control system failure

Terminate the mission.  The safety pilot takes manual control and lands the UAS.

Engine failure

The safety pilot takes manual control.  Maneuver to as safe an area as possible, land vertically.

Magnetometer failure

Terminate mission.  The safety pilot takes manual control and lands the UAS.

Artificial horizon failure

Terminate the mission.  The safety pilot takes manual control and lands the UAS.

Poor GPS reception

Terminate the mission.  The PIC commands the UAS to return to the recovery point and auto land.

GPS failure

Terminate the mission.  The safety pilot takes manual control and lands the UAS.

Lost Communications PIC to observer

See lost communications procedures.

Lost communications PIC to ACT

See lost communications procedures.