Control Station:

The Rotomotion UAS ground control station (GCS) is a computer software program that runs on any modern computer running either the Windows XP or Linux operating system.  The GCS displays the telemetry from the UAS and provides a way to monitor and command the UAS.  The GCS displays the UAS location on a map that has been geo-referenced so that locations on the map equal actual locations on the earth.  By clicking points on the map, waypoints can be established that the UAS will fly to during a mission.  At any time the UAS pilot may point to a map location and see the latitude and longitude under the mouse pointer.  The GCS also receives and records video from the UAS.  During flight the pilot, by means of a joystick connected to the GCS computer, can pan, tilt or zoom the video camera on board the UAS.  A joystick connected to the GCS computer can be used to maneuver the UAS if necessary.

The GCS sends the flight profile to the UAS prior to flight.  The UAS is then capable of fully autonomous flight.  During flight the GCS sends data to the UAS once every second to test the control link availability.

The North Little Rock Police Department will purchase two complete GCS packages.  During flight operations only one GCS will be operational at any one time.