Communication Systems Description

UAS Communications systems

Primary safety controller

The primary safety controller is a standard radio control (R/C) aircraft controller, Spektrum model DX7, operating on the FCC approved 2.4 GHz unlicensed band.

Video downlink with data carrier

The Rotomotion UAS will use the 2.4 GHz unlicensed band for video downlink communications.  Several channels are available and any of the available channels may be selected prior to flight.  The video downlink operates with a radio frequency amplifier at or below the maximum output power allowed by the FCC for unlicensed operations.

Command and control uplink

The UAS will utilize the unlicensed 900 MHz band.  Several channels are available and any of the available channels may be selected prior to flight.  The command and control uplink operates with a radio frequency amplifier at or below the maximum output power allowed by the FCC for unlicensed operations.

Full duplex digital link

The UAS can utilize the unlicensed 2.4 GHz band or the unlicensed 5.8 GHz band to carry video and command and control data.   This link operates with a radio frequency amplifier at or below the maximum output power allowed by the FCC for unlicensed operations.

All UAS communications systems use radio equipment meeting FCC requirements for operations in the bands used.

Observer/Pilot Communications systems

Direct communications between the UAS pilot (PIC) and observer will be provided during all flight operations.  Primary communications will be by use of 800 MHz trunked radio equipment operating on a FCC licensed public safety system.  Secondary communications will be by use of a FCC licensed non-trunked 800 MHz conventional repeater using the same radios as the primary communications system.  Backup communications will be by use of department issued cell phones.

ATC Communications

The PIC will have aircraft radios tuned to the local ATC frequency, and the guard frequency (121.5 MHz).  Backup communications will be by use of department issued cell phones.