Control Station:

The Rotomotion UAS ground control station (GCS) is a computer software program that runs on any modern computer running either the Windows XP or Linux operating system.  The GCS displays the telemetry from the UA and provides a way to monitor and command the UA.  The GCS displays the UA location on a map that has been geo-referenced so that locations on the map equal actual locations on the earth.  By clicking points on the map, waypoints can be established that the UAS will fly to during a mission.  At any time the UAS pilot in command (PIC) may point to a map location and see the latitude and longitude under the mouse pointer.  The GCS also receives and records video from the UA.  During flight the PIC, by means of a joystick connected to the GCS computer, can pan, tilt or zoom the video camera on board the UA.  A joystick connected to the GCS computer can be used to maneuver the UAS if necessary.

The GCS sends the flight profile to the UA prior to flight.  The UA is then capable of fully autonomous flight.  During flight the GCS sends data to the UA once every second to test the control link availability.

The North Little Rock Police Department has purchased two complete GCS packages.  Both are running on Panasonic Toughbook computers using a Linux operating system.  During flight operations only one UA will be controlled by the GCS.  One GCS computer is used as the PIC’s control station.  The second GCS computer is utilized as the camera control station and is operated by a camera operator.  This leaves the PIC free to devote all his attention to operating the UAS.