Our ground station communicates with the YellowTail aircraft via 900 MHz serial modem link.  This link enables ground station operator(s) to monitor real-time vehicle data and provide autopilot configuration commands.  The data communications link is not safety-critical, however, since our R/C pilot can override the autopilot at any time through a separate 72 MHz R/C link.

We propose operation below 400’ AGL in Class G airspace thus do not anticipate the need for persistent (direct-to-pilot) communication with air traffic control unless required for CoA approval.

All support personnel, including pilot, observer, and ground station operations will be co-located on the ground proximal to but outside the flight operations area.  These personnel will be able to speak directly to each other in most situations.  We utilize hand-held radios to enable the observer, physically adjacent to the pilot, to communicate with ground station operator(s) should they be in different areas.