Our lost link procedure is captured below via a brief hazard analysis including methods by which the flight can be terminated in the event of lost link or failure.  In short, whenever the R/C link is lost, the PCM receiver will execute a “ditch” flight termination maneuver.  The 900 MHz data link does not carry pilot or autopilot commands and can be overridden by the R/C pilot.

Flying Fish Hazard Analysis & Flight Termination System (FTS) Specification:

We will operate such that the pilot maintains ultimate backup authority at all times through a COTS Servo Switch Controller sold by Microbotics, Inc.  Given our operation over uninhabited open water, the primary hazard imposed by Flying Fish is to transit out of the CoA flight area.  As is standard practice for safety-critical systems, we use backup systems to limit (i.e., bring to within acceptably low probability) the possibility that Flying Fish will exit its flight area.  This event that brings risk only when operating as a UAS, not while drifting passively on the water.

We use a fault tree to validate flight operations safety, supplemented by continuous observation of the flight operations area to verify it is uninhabited.  Figure 1 shows a fault tree associated with the “transit out of designated flight area” event.  Three events simultaneously must occur for this transition:  1) Loss of autopilot control, 2) Loss of R/C control, and 3) Failure of the motor controller auto-shutdown systems activated when a proper PWM signal is no longer received.  If the flight must be terminated, either the R/C pilot can manually land or the ground station operator can issue a “land immediately” (in glide) or “ditch” command.  The motor controller auto-shutdown would only be activated in the case that the COTS Servo Switch Controller itself failed, preventing either R/C pilot or autopilot commands from properly reaching the motors and servos.  This provides a third flight termination option, since Flying Fish operates at low altitudes thus has the relatively poor glide ratio expected from a float plane.  This redundant design, supplemented by a standard PCM receiver that commands a power-off ditch into the water whenever the R/C link is fully lost, enables safe handling of any single failure and key multi-system failures.

The Flying Fish pilot, observer, and support personnel monitor the flight from a support boat/ship with a continuous line-of-sight view from just outside the flight operations area.  Following standard AMA protocol, Flying Fish never flies over or toward the support ship, thus can be safely ditched as needed.

Figure 1:  Top-level Fault Tree.