The Funtana Control Station, shown in Figure 1, includes a ground station laptop running Linux, an extended-range COTS modem for 115Kbaud RS/232 serial communication with the aircraft over a 900 MHz wireless link, battery power system, and an R/C transmitter enabling the pilot in command to manually fly the aircraft at any time.  The ground station runs a standard heads-up display (HUD) graphical user interface (GUI) developed with the QT protocol interfaced with C-based communication, data processing, and data logging functionality.  Waypoint and real-time position data is overlaid on a Google Earth satellite image of the operating area.  An onboard autopilot can fly the vehicle autonomously.  An R/C pilot can fly remotely via direct LOS visual observation of the aircraft.  Displayed data is based on the real-time data feed from the aircraft and is used for mission monitoring and command uplink purposes only.

Figure 1:  Control Station Components in their Travel Case (modem antenna not shown).