YellowTail Phase I Vehicle Specifications

University of Michigan

The YellowTail Phase I vehicle is fully constructed and ready to fly. Shown in Figure 1, the aircraft is a small, single electric engine, high aspect ratio, unmanned vehicle. The fuselage is constructed almost entirely of fiberglass with a carbon fiber boom connecting the tail and the fuselage. The wings are constructed of foam covered by fiberglass with two carbon fiber spars running through each wing. The main battery, motor, and motor controller are all mounted within the fuselage. Secondary battery banks are mounted inside the wings along with the avionics.  The control surfaces are actuated with R/C servos.  The tail servos are mounted in the fuselage, and the wing servos are mounted in the wings adjacent to the control surfaces. In order to simplify transportation, the outer tapered sections of the wing can be removed while the rest of the plane remains permanently assembled.  The solar panels are mounted on the surface of the wings, and are removable, to allow flight without solar energy harvesting.  Physical design specifications for this airframe are provided below. Specifications for the onboard avionics system are also provided.

Figure 1:  YellowTail Phase I Platform.

Physical Design Specifications:

General:

Flight Weight: 3.6 kg

Cruise Speed: 50.4 kph (14 m/s)

Estimated Endurance: 24+ hours

Estimated Range: 1200+ km

Propulsion:

Configuration: Single, Tractor

Motor: Multiplex BL480-6G (brushless in-runner, geared, 4.4:1 gear ratio)

Speed Control: Castle Creations, Phoenix-36 (36A brushless)

Propeller: Graupner CAM 16x10, carbon reinforced nylon

Thrust: 14 N  (3.15 lbs)

Thrust/Weight: 0.4

Battery Type: 4-cell Lithium-Polymer, 14.8V (average), 16.8V (peak)

Battery Capacity (nominal): 300 Amp-Hours

Propulsion Notes: The Battery Capacity can be augmented by solar energy harvesting when the solar panels are on board.

Main Wing:

Airfoil: NACA 4414 (inboard), NACA 4412 (outboard)

Span: 3.1m

Chord: 23cm (root), 6.9cm (tip)

Dihedral: 0deg at root, 10deg starting 0.33m inboard from tip

Wing Area: 0.56m2

Wing Loading: 6.43 kg/m2

Aileron Span: 35.5cm

Aileron Chord: 5.2cm

Wing Configuration Notes: Constant chord out to 0.6m, constant taper for remaining span

Horizontal Tail:

Airfoil: Flat plate

Span: 55cm

Chord: 13cm (root), 11.5cm (tip)

Area: 0.07m2

Tail/Wing Separation:  73cm

Elevator Chord: 3.2cm

Elevator Configuration Notes: Full span elevator

Vertical Tail:

Airfoil: Flat plate

Height: 26cm

Chord: 16cm

Area: 0.028m2

Stabilizer Separation: 87cm

Rudder Chord: 8cm

Rudder Configuration Notes: Full height, cut in at base to accommodate elevator

RF Equipment:

Radio Control System:

- Rx: Futaba, R149DP, 9-channel PCM dual-conversion receiver

- Frequency: 72.970 (Ch. 59)

Computer Uplink:  Tx/Rx: Digi, XTend XT09-PKI, serial

- Frequency: 900MHz

- Power: less than 1.0W

Vehicle Avionics System:

The onboard avionics system design is illustrated in Figure 2.  A Microbotics MIDG-II inertial navigation system (INS) provides 6-DOF filtered state estimate from a set of three-axis gyros, accelerometers, and magnetometers in combination with GPS.  The sensor system also includes a pressure transducer for redundant airspeed measurements.  A Gumstix 400MHz embedded Linux computer provides onboard computational power, working in concert with an Atmel-based Robostix expansion card.  The integrated system provides analog-to-digital conversion, multi-port serial communications, 802.11b Wi-Fi (active only in the lab for high-speed data transfer), and large-volume data storage via onboard Compact Flash.  A Microbotics Servo Switch Controller provides a fail-safe switch between computer and radio-controlled pilot servo commands as well as a means for the computer to log pilot inputs.  Long-range communication and ground station telemetry are enabled with a Digi XTend 900MHz radio modem.  The embedded Linux computer runs C software for data acquisition, guidance, navigation, control, and communication with the ground station.   The Atmel microprocessor communicates with the Servo Switch Controller and acquires data from the analog pressure sensor as well as monitoring battery voltages.  The Atmel communicates with the Linux processor via an embedded I2C protocol.

Figure 2: YellowTail Phase I Avionics System.