Funtana X100 Vehicle Specifications

University of Michigan

The Funtana X100 platform is a ready to fly commercial-off-the-shelf (COTS) hobby airframe. Shown in Figure 1, the airframe is an electrically-powered aerobatic aircraft. The airframe is constructed of reinforced plywood with minor non-structural modifications made to support our custom avionics.  Small air pressure lines are routed through the inside of the wing and connected to small metal pressure ports on the wing surface. Our power system and avionics boards are securely mounted inside the main fuselage.  Also attached to the right wing is a probe that measures angle of attack and sideslip angle, along with a Pitot probe.  Physical specifications for this airframe are provided below. Specifications for the onboard avionics system are also provided.

Figure 1:  Funtana X100 Platform.

Physical Design Specifications:

General:

Flight Weight: 5.00 kg

Cruise Speed: 32.4kph (9m/s) (Max Speed: 64.8kph (18m/s))

Estimated Endurance: 10 minutes (full throttle), 16 minutes (cruise)

Estimated Range: 14.4 km

Propulsion:

Configuration: Single, Tractor

Motor: E-flite, Power 110-295kV (brushless out-runner, direct-drive)

Speed Control: Castle Creations, Phoenix-85 (85A brushless)

Propeller: APC 19x8, carbon reinforced nylon

Thrust: 84.5N (19.0 lbf)

Thrust/Weight: 1.72

Battery Type: 8-cell Lithium-Polymer, 29.6V (average), 33.6V (peak)

Battery Capacity (nominal): 4150mAh

Main Wing:

Airfoil: NACA 0015

Span:  1.77m

Chord: 36.5cm (root), 17.0cm (tip)

Dihedral: 0deg

Wing Area: 0.715m2

Wing Loading: 6.99 kg/m2

Aileron Span: 73.1cm (9.7cm average outboard from root)

Aileron Chord: 6.03cm (average)

Flap Span: N/A

Flap Chord: N/A

Wing Configuration Notes: Straight tapered

Horizontal Tail:

Airfoil: Flat plate

Span: 30.6 cm

Chord: 11.4 cm (average)

Area: 265cm2

Tail/Wing Separation: 0.939m

Elevator Chord: 7.1cm

Elevator Configuration Notes: Over-sized elevator

Vertical Tails:

Airfoil: Flat plate

Height: 36.3cm

Chord: 31.1cm (root), 14.6cm (tip)

Area: 160.8 cm2  (stabilizer), 598 cm2 (total)

Rudder Chord: 10.2cm (average)

Rudder Configuration Notes: Over-sized rudder

Landing Gear:

Separation: 23.5 cm (on center)

Distance Below Wing: 15.5cm (wing bottom to tire top)

RF Equipment:

Radio Control System:

- Tx: Futaba 9C 9-channel FM Transmitter

- Rx: Futaba, R149DP, 9-channel PCM dual-conversion receiver

- Frequency: 72.970 (Ch. 59)

Vehicle Avionics System:

The onboard avionics system design is illustrated in Figure 2. A Microbotics MIDG-II inertial navigation system (INS) provides 6-DOF filtered state estimate from a set of three-axis gyros, accelerometers, and magnetometers in combination with GPS. The sensor system also includes seven pressure transducers for airspeed, angle of attack, sideslip angle, and pressure distribution along the top wing chord surface. A Microbotics Servo Switch Controller (SSC) provides a fail-safe switch between computer and radio-controlled pilot servo commands as well as a means for the computer to log pilot inputs. A Diamond Systems Athena II PC/104 computer running the QNX Neutrino real-time operating system provides onboard flight management, datalink, and data acquisition capabilities. The Athena II's data acquisition includes analog-to-digital conversion and serial communications for the INS and SSC. Embedded C software performs data acquisition, guidance, navigation, and control. The Athena II also has an Ethernet port for program and data transfer to an external computer, although in flight a serial RS/232 modem is the exclusive datalink. The Athena II has an onboard flash storage capability.

Figure 2: Funtana X100 Avionics System.