Our ground station communicates with the Funtana X100 aircraft via 900 MHz serial modem link. This link enables ground station operator(s) to monitor real-time vehicle data and provide autopilot configuration commands. The data communications link is not safety-critical, however, since our R/C pilot can override the autopilot at any time through a separate 72 MHz R/C link.

We propose operation below 400’ AGL in Class G airspace thus do not anticipate the need for persistent (direct-to-pilot) communication with air traffic control unless required for CoA approval.

All support personnel, including pilot, observer, and ground station operations will be co-located behind the flight line at the AMA field and outside the flight operations area.  These personnel will be able to speak directly to each other in most situations.  We utilize hand-held radios to enable the observer, physically adjacent to the pilot, to communicate with ground station operator(s) should they be in different areas.