The Flying Fish platform supports two wireless communication links with the ground:  a 72MHz R/C transmitter/receiver link approved for use with model aircraft, and a 900MHz RS/232 serial data link.  A short-range 2GHz wireless Ethernet connection is present but powered down during flight operations.

The Futaba 72MHz transmitter and onboard Futaba PCM receiver allow the pilot to manually fly the seaplane at any time using standard R/C aircraft controls.  COTS R/C transmitter and receiver antenna systems provide sufficient range for our watch circle maintenance application, verified before each flight through surface range checks where multipath effects (water surface reflection) minimize range.

The onboard computer system communicates with the ground station laptop with a pair of Digi International XTend 900MHz spread spectrum modems, configured to a communication link rate of 115Kbaud.  The onboard lightweight antenna is provided by the manufacturer.  A larger ground or ship-based antenna maximizes ground station sensitivity to incoming data.  All transmission power levels are less than 1 Watt.