University of Wisconsin-Madison Department of Civil and Environmental Engineering Radio-Controlled Airplane Use

Field Site Emergency Contacts*

Allequash Site:

Minocqua Fire Department (Chief Andy Petrowski) – (715) 356-3234 or (715) 356 5296

Minocqua Police Department (Department Head Norbert McMahon) – (715) 356 3234 or (715) 356 1482

Marshfield Clinic in Minocqua and Howard Young Medical Center in sister city of Woodruff provide well-regarded rural medical care in the event of an emergency

Pecatonica Site:

Barneveld Fire Department (Fire Department Chief Jeff Jenson) – (608) 924 9487

Barneveld Police Department (Chief of Police Brian Schneider) – (608) 924 6861

Barneveld Rescue Squad (Squad Head Al Wright) - (608) 924 2852

*Flight locations have cell phone reception

Emergency Procedures

- Observer(s) come in contact with flight vehicle/flight vehicle propeller

- Pilot immediately terminates flight operations and lands/moves vehicle to a safe position (depending upon the situation)

- Disconnect motor battery packs

- All individuals seek emergency medical attention (contacts above) and provide basic first-aid care to those in need

- Loss of control of flight vehicle/vehicle failure

- Pilot notifies all observers of the situation

- Observers move to a safe location out of way of flight landing operations and the flight course

- Pilot attempts to regain control of aircraft and performs emergency landing

- Pilot lands vehicle to assess for safety concerns

- Flights do not reoccur until damage is assessed and repaired

- If pilot loses control of aircraft and it flies out of visual range, the sheriffs office will be contacted immediately so that local airtraffic authorities can be notified.

- Fire aboard flight vehicle or in the area of vehicle operations

- Pilot or observer(s) notifies the others of the situation

- Observer gets fire extinguisher

- Pilot brings flight vehicle to a safe location

- Observer phones local fire department if fire cannot be contained by use of the fire extinguisher

- Test personnel move 25 yards from the fire scene