Standard Operating Procedures for University of Wisconsin-Madison Department of Civil and Environmental Engineering Radio-Controlled Airplane Use

All Academy of Model Aeronautics Safety Code guidelines will be followed with the exception of the 400 foot ceiling.  (1000 feet will not be exceeded)  In addition, the following procedures will be used.

Safety Equipment/personnel

1)	First-Aid kit

2)	Fire extinguisher

3)	Trained EMT personnel

4)	Working cell phones to contact emergency officials

Pre-flight procedures

1)	Visual inspection of all flying surfaces, hinges, control links, servos, wing 	supports, landing gear, propeller, camera mounts, etc. by pilot.

2)	Visual inspection of above by secondary observer.

3)	Visual inspection and voltage reading of flight batteries.

4)	Thorough range check of R/C equipment.

5)	Camera equipment check.

6)	Observation of weather

1) 	Steady winds under 12 mph and gusts under 15 mph

2)	No thunderstorms in area

In-flight emergency procedures

1)	Follow Hazard Analysis and Flight Safety procedures

Post-flight procedures

1) 	Turn off all radio equipment and disconnect flight batteries.

2)	Check batteries for voltage and temperature to ensure safety.

3)	Remove propeller from aircraft before storage.

4)	Visual inspection of aircraft for damage.