Hazard Analysis for University of Wisconsin-Madison Department of Civil and Environmental Engineering Radio-Controlled Airplane Use

| Operations and Equipment Problems/Failure | Cause | Potential Impact(s) | Countermeasure(s) |
| --- | --- | --- | --- |
| Loss of Radio Uplink | Flight out of range of radio system; malfunction of transmitter or receiver; radio wave interference | Loss of or impaired control of flight vehicle; potential for impacts to cause damage to vehicle or injure observers | Range check/analysis as part of pre-flight testing; post-test flights occur only within the range of testing; check for frequency interference in flight areas; full charge of batteries prior to flight |
| Control System Malfunction | Damaged servo, control link failure, hinge failure, control surface failure | Loss of or impaired control of flight vehicle; potential for impacts to cause damage to vehicle or injure observers | Use only proven, high quality components in control system; thorough pre-flight check prior to every flight; pilot proficiency in emergency control situations |
| Fire | Overheating of batteries | Loss of flight vehicle; loss of control of flight vehicle; injury to bystanders; fire hazard for area vegetation | Use of low voltage cutoff devices; preflight inspection of batteries; make a fire extinguisher available during all flight operations |
| Airplane/flight vehicle contact with observer(s) | Loss of control of vehicle due to fire, loss of radio uplink, etc. | Bodily injury/harm to flight observers | Fly only in ideal weather conditions (winds not greater than 12 mph); pilot proficiency in flight maneuvers (power off landings, etc.); first-aid kit available whenever flight vehicle is in use |
| Propeller contact with flight observer(s) | Close proximity of flight observers to flight vehicle; loss of control of flight vehicle | Bodily injury/harm to flight observer(s) | Observer(s) kept at safe distance from flight operations (take off and landing); briefing of flight observer(s) on operational safety procedures and hazards; first-aid kit available whenever flight vehicle is in use |
| Ejection of material from the flight vehicle | Structural failure of vehicle related to environmental stress, system failure, etc. | Bodily injury/harm to observer(s) | Preflight inspection of flight vehicle; availability of personal protective equipment (PPE) including safety glasses |
| Damaged flight vehicle | Contact with ground or trees during a rough/windy landing | Structural failure to vehicle which could result/promote vehicle malfunction | Fly only in ideal weather conditions (winds not greater than 12 mph); pre and post-flight inspection of flight vehicle to look for structural abnormalities |
| Loss of engine/engine malfunction | Diminished motor battery, battery failure or damaged vehicle power system | May result in a forced, potentially harmful landing | Pilot proficiency in power-off landings; pre and post-flight inspection of vehicle engine and structure |
| Ejection of material from flight vehicle propeller | Structural failure or ground contact resulting in a deficiency | Bodily injury/harm to observer(s); potential for eye injuries | Availability of personal protective equipment (PPE) including safety glasses/goggles; observers kept at safe distances from flight vehicle when propeller is in motion |
| Test environment | High grass, vegetations, water | May require an aborted takeoff or necessitate a rough landing | Mow runway/keep runway area free of potentially hazardous debris; ensure pilot is knowledgeable about flight area geography |

09/04/2007 v.1.0