Lost Link and Lost Communication Procedures for University of Wisconsin-Madison Department of Civil and Environmental Engineering Radio-Controlled Airplane Use

The likelihood of a loss of radio control will be minimized through range check/analysis as part of pre-flight testing.

In the event of a loss of radio uplink, the pilot will attempt to regain control by altering the direction of antennas and other means and make an emergency landing while observers follow procedures in the safety procedure plan.

If control cannot be regained, the aircraft uses a fail-safe receiver.  This is an off-the-shelf product made by Futaba, a recognized leader in the RC industry.  When it senses a loss of signal, it automatically sets the controls to pre-determined settings.

Our Fail-Safe settings are as follows:

1)  Throttle 0% to stop the motor.

2)  Full right rudder to ensure the aircraft does not travel beyond test area.

3)  Slight down elevator to force the aircraft into a shallow descent.

The aircraft is highly visible and can be seen from a great distance, it is very lightweight so it flies very slowly, and it has only enough flight batteries to power the motor for ~10 minutes.  If all fail-safe features fail, the aircraft will be tracked from the ground and every possible measure will be taken to keep it in sight until it runs out of batteries and lands.

Loss of video downlink is not a concern as it will not be used for aircraft operation or navigation (only data collection).