Launch/Recovery of the BAT 3

Launch

The BAT 3 is launched off a truck-mounted bungee-powered catapult. An autonomous launch requires no pilot involvement. Once the engine is started and the aircraft is ready to go, the R/C transmitter is switched to Autonomous mode, which causes the throttle to automatically advance to full. The flight computer then waits to detect the launch acceleration. Upon detecting the launch, the flight computer initiates a climb out sequence up to 100’ prior to initiating normal waypoint navigation to waypoint 0.

A manual launch requires a pilot to manually fly the aircraft for takeoff. Upon reaching a

safe altitude the R/C transmitter can be switched to Autonomous mode and the flight

computer will initiate normal waypoint navigation to waypoint 0. The normal launch method is autonomous.

Recovery

The BAT 3 can be landed either autonomously or manually via RC control. For autonomous landing: a landing flight plan is uploaded, consisting of an approach waypoint 0.25 miles away with a maximum 12 degree glide slope, a second point at 0 elevation and a third point 100 ft further 50 ft below the ground. As soon as the BAT lands, the RC pilot takes over manually and kills the engine. Manual landing: the BAT is brought close to the home location, and the RC pilot takes over manually and lands the aircraft.