ImageAire Lite ii-2 System Summary

February 3, 2009

Description

The ImageAire Lite ii-2 is an Unmanned Aircraft System (UAS) designed to carry multiple imaging systems in a small, hand-launched, electric powered platform.  The vehicle/science system is designed to be easily operated by a proficient radio control model pilot.  The imagery from the 3 onboard cameras is recorded onboard to prevent data dropouts, while the nadir view color camera with GPS navigation data overlay is downlinked for real-time viewing and post-flight analysis.  The UAS is designed for hand launch, requiring no special runway to launch or recover, using skids (or optional wheels) for landing.

UAV Airframe

The ImageAire Lite ii-2 airframe is based on commercially available Almost Ready-to-Fly (ARF) radio control model airplane kits.  After determining that the weight, wing loading, and fuselage volume was adequate, the airframes were modified for electric propulsion and payload systems installation.  The wing was extended by joining two wings, and each fuselage contains its own independent PCM control system.  Additional vertical tail area was added via dorsal fins to increase directional stability.

The ImageAire Lite ii-2 UAS was successfully test flown on December 18, 2008 at the Wallops Flight Facility UAV Runway.  The UAS was flown without wheels at a weight of 6.4 pounds, with the CG was located at the nominal location of 2.7” aft of the wing leading edge, or at 27% as calculated by area (30% of the root chord).  Vehicle performance was excellent during the 16.5 minute flight, exhibiting adequate power and climb with good stability and control in all axes.  The payload performed nominally, gathering data onboard and via downlink.

Dimensions

Wingspan			126”

Wing Area			1101 in2

Wing Chord			9” at root, 6” at tip

Wing Aspect Ratio		14.4:1

Fuselage(s) Length		40”

Fuselage Width & Height	2.5”W x 4”H

Tail Configuration		Twin booms, twin tails

Horizontal Tail Span		2 stabs, each 22”

Horizontal Tail Area		2 stabs, each 100 in2, 200 in2 total

Vertical Tail Height		2 verticals, each 7”

Vertical Tail Area		2 verticals, each 55 in2, 110 in2 total

Weights

Flight Weight			6.4 lbs. (6.53 lbs. w/wheels)

Wing Weight			1.55 lbs.

Fuselage(s) Weight		(2) 1.55 lbs. each

Payload Pod			1.75 lbs.

Propulsion

Motors				(2) electric AXI 2212/26 brushless “outrunner” motors

Controllers			(2) Electrifly SS-25 BEC brushless controllers

Batteries			(2) Electrifly 11.1v/2100mah lithium-polymer batteries

Control Systems

Radio				Futaba 6EXAP PCM transmitter – 72.270 MHz/Ch 24

Receivers			(2) Futaba FP-R148DP receivers – 72.270 MHz- Ch 24

Servos				(4) Hitec HS-81MG micro metal gear servos

Performance

Stall Speed			15 mph

Operational Speed		20-35 mph

Normal Cruise			20-25 mph

Slant Range, maximum	2000 ft

Endurance, maximum		30 minutes (no reserve)

Altitude, max			2000 ft

Launch and Recovery		hand launch, ground recovery

Center of Gravity

The center of gravity is indicated by a stripe located on the wing center section, denoting a CG location of 2.5” to 2.875” from the wing leading edge.  This translates to a 25-30% CG range as measured by wing area, or 27-32% of the (9”) root chord.  As provided, the CG falls in the center of this range.

Payload Systems

The ImageAire Lite ii-2 UAV payload instrumentation is contained in a pod suspended from the center of the ImageAire Lite ii-2 wing.  This design isolates the payload electronics and power systems from the UAS control and propulsion systems.  The Payload Systems consist of the following:

1	Sony Color Microvideo Camera, 6mm lens, nadir view

1	Sony Monochrome Microvideo Camera, 6mm lens, 695 nm NIR filter, nadir view

1	FLIR Photon Thermal Infrared Camera, 19mm lens, nadir view

3	Micro Digital Video Recorders (DVR’s)

1	2.4 GHz Video Transmitter (FCC Part 15 compliant)

1	Miniature GPS Receiver w/Integrated Antenna

1	GPS Video Overlay Board

1	Madgetech PRHTemp101 Datalogger

1	Great Planes 3-cell/11.1V/910mAh Lithium-Polymer Battery

Imagery from the 3 video cameras is recorded onboard and downloaded after flight.  The color camera is also downlinked via the 2.4 GHz video transmitter to provide the scientist with real-time data.  This imagery may be recorded on the ground and may include GPS video overlay if desired.  The transmitter is set to channel 4 (2470 MHz) to prevent interference from Wi-Fi devices, and the receiver will need to be set to channel 4 as well.