ImageAire Lite ii-2 UAV Operating Procedures (no ailerons)

January 22, 2009

# T-1 Day

1) Charge Batteries:

Vehicle (Control and Propulsion)

Control Transmitter

Sensor Systems (as required)

2) Measure Weight and Center of Gravity

3) Preflight Checks:

Sensors

Data Systems

Vehicle Systems

# T-90 Minutes

4) Preflight Planning Session:

Mission Objectives Flight Profile, and Contingencies

Vehicle/Systems Status

GO/NO GO Criteria

# T-1 Hour

5) Complete Battery Charging:

Vehicle (Control, Propulsion, and Sensor/Data Systems)

Control Transmitter

6) Preflight Checks:

Sensors

Data Systems

Vehicle Systems

7) GO/NOGO Decision

Range Communications are GO

# T- 30 Minutes

8) Transport Vehicle and Support Equipment to Flight Area

9) Record Meteorological Conditions:

Wind Speed and Direction

Temperature and Humidity

Barometric Pressure

10) Confirm Operations Area is Clear of Non-participating Personnel

11) Inspect:

Propellers and Motors

Wings and Attachments

Tail and Control Surfaces

Sensor Systems

# Preflight, Launch and Flight

12) Final Vehicle Preparation:

Restrain vehicle

Clear propellers

Plug in propulsion batteries

Secure hatch

13) Sensor Systems ON (unless Payload Sensors/Instrumentation are to be installed later)

14) Transmitter ON

15) Receiver ON

16) Control Check:

Elevator UP

Elevator DOWN

Rudder LEFT

Rudder RIGHT

Motor ON

Motor OFF

Confirm Control Mixes (as applicable)

17) Fail-Safe/Interference Check:

Motor SLOW

Transmitter OFF

Confirm Fail-Safe control position (rudder LEFT, elevator UP, motor OFF)

Confirm no additional un-commanded control surface movements

Transmitter ON

18) Range Check:

Verify Transmitter Antenna collapsed

Relocate Transmitter 100 Feet away from Vehicle

Repeat Control Check – Step 16

Repeat Fail-Safe/interference check – Step 17

19) Confirm Transmitter Antenna is EXTENDED

20) Confirm Payload Sensors/Instrumentation are enabled for flight.

21) If Payload Sensors/Instrumentation are not ready for flight, power down Vehicle Control and Propulsion Systems.  Install Payload Sensors/Instrumentation and repeat steps 12 thru 20.

22) Confirm Launch Direction and Flight Pattern

23) Confirm Pilot and Observer are GO

24) Confirm Launch Area is clear and announce “Launching”

25) Motors ON and Launch (into wind - if possible)

26) Climb and Maneuvering Flight

27) Research Measurements and Data Acquisition

28) Confirm Landing Area is clear and announce “Landing”

29) Land Vehicle (into wind – if possible)

30) Receiver OFF

31) Transmitter OFF

32) Safe Vehicle: Open Hatch and Unplug Propulsion Batteries

33) Sensor Systems OFF

# Post Flight

34) Announce “Operations are Complete”

35) Inspect Vehicle

36) Review Sensor Data (as required)

37) Record Flight Log

Note: For Rapid Flight Termination - a Descending Spiral for an estimated descent rate of  ~500 fpm will be commanded