2.  COMMUNICATIONS SYSTEM

The Communications System consists of three components: 1) Imaging Payload, 2) Video Systems Ground Station, and 3) Pilot – Observer verbal communication.

All Components are described in detail below:

IMAGING PAYLOAD SYSTEMS:

The imaging payload systems are comprised of real-time selectable color and monochrome thermal infrared cameras with video transmission to a ground station. The NTSC format video signal from both cameras feed a switch controlled by the payload RC system, enabling a ground based operator to select the video to be transmitted via a 2.4 GHz transmitter.  This transmitter is FCC Part 15 compliant and requires no special license to operate. Switching between the visible and thermal infrared cameras enables comparison of specific features. The downlink video signal will be captured using a 2.4 GHz video receiver and recorded on a Sony Digital 8mm portable VCR for storage and analysis.

VIDEO SYSTEM GROUND STATION

The ImageAire iii-2 video system ground station consists of a 2.4 GHz video receiver and a Sony Digital 8mm VCR.  After charging batteries, connect the video output of the video receiver to the audio/video input jack on the Sony VCR using the supplied adapter and extension cable.  Turn both units ON, set the video receiver to channel 1 (2401 MHz) using the button on the side of the enclosure, insert tape in VCR, and observe video signal on the VCR LCD monitor.  When the Pilot is ready to launch the aircraft, the Payload Operator should start recording by pressing the RECORD button and the button immediately to the right simultaneously.  Use the payload control RC transmitter to switch video sources as desired while observing the LCD monitor.  When recording is complete, press STOP to halt recording.

Video data may be analyzed post-flight by using a computer with a video capture card to “grab” visible and infrared images of ground features for comparison.

The maximum operating range of this system is approximately 2000 feet, which roughly corresponds to normal line-of-sight radio control flight.  An occasional video signal dropout is to be expected and should not affect data analysis.  If the plane is too far away to get a good video signal it is probably too far away to reliably control.

PILOT/OBSERVER COMMUNICATION

There is an observer standing within 10 ft of the pilot for the duration of the launch, flight and recovery.

- Pilot prepares for launch.

- Observer gives the signal that all is clear for launch.

- Pilot gives the command to count-down for launch.

- Observer counts down and hand launches aircraft into the wind.

- Observer directs pilot over the experimental area and warns pilot of any incoming aircraft or birds.

- Observer gives the command to end data collection and signals all is clear for landing.

- Pilot lands the aircraft as soon as it is safe to do so.