Communication systems for BAT 3

The diagram below shows the BAT 3 communication systems.

Two-way communication between the aircrafts’ flight computer and the ground station is over the 900 MHz modem. After the initial start up sequence has been completed, this communication link is not essential to maintain flight. The aircraft can operate independent of the groundstation, so there is no immediate problem if communication is lost or the ground station fails. However, the two-way link is required to re-task the aircraft and receive flight status updates.

Frequencies:

Video:

1 Watt EIRP, Antenna is a zero db, fixed frequency (user configurable before

flight): 2.410, 2.430, 2.450, 2.470, 2.413, 2.433, 2.453, 2.473GHZ

Manufacturer: Black Widow AV, Broomfield, CO

Telemetry:

1 Watt EIRP Antenna is 0db, spread spectrum 902 to 928 MHZ, 400 KHZ

increments

Manufacturer: Microhard, Calgary, Canada, FCC ID#NS902P5

Radio Control back up link:

3/4 Watt EIRP Antenna is 0 db, standard model aircraft frequencies, such as

72.490 MHZ

Manufacturer: JR Radio Control, Japan, FCC ID#BRWNET72FP