ImageAire (iii-2)

USDA Configuration

Thermal/Visible Imaging UAV System

NASA Goddard Space Flight Center

Instrumentation Sciences Branch

Wallops Island, Virginia

January 26, 2005

The ImageAire iii-2 is a thermal and visible spectrum imaging Uninhabited Aerial Vehicle (UAV) system. The vehicle/science system is designed to be easily operated by a team of two: a proficient radio control (RC) Pilot and a Payload Operator. The payload operator can monitor the downlinked data and perform payload control functions via a second radio control system. The hand-launched, electric powered vehicle requires no special runway to launch or recover and uses skids for landing. The UAV operational weight is approximately 8 pounds, with a fuselage width that will enable an easy hand launch.  Every effort has been made to make this system complete, including portable power and data systems for operations at remote sites.

The ImageAire iii-2 airframe is based on a commercially available RC model airplane kit.  Modifications include electric propulsion and payload systems installation.  The wing was modified by adding tip extensions to provide additional lift and is attached using two ¼” nylon screws.  The Center of Gravity (CG) should be checked after any modifications by balancing the plane on your fingertips.  It should balance at or slightly behind the 25% mark on the underside of the wing.  The final weight and CG of the ImageAire iii-2 as provided was 7.84 pounds at 27 percent chord on December 2, 2004.

CG MUST BE LOCATED BETWEEN 25% AND 30% OF THE WING CHORD!

Power SYSTEM

Two packs of lithium polymer batteries are used to provide power for both propulsion and onboard power in ImageAire iii-2.  Each pack consists of lithium polymer cells arrayed is a 3-parallel/3-series configuration to provide a nominal value of 11.1 volts under load with a capacity of 6 amp hours each.  These packs are connected in parallel to provide a total of 12 amp hours at 11.1 volts.  This configuration provides an extremely high energy density, but requires some care.  If the battery voltage drops below 3 volts per cell, or 9 volts for the packs, the batteries can sustain permanent damage.  Please note that even with all systems switched off, the speed control still draws a small amount of current, which will drain the batteries over time. Always unplug batteries after use to preclude damage.

DO NOT DISCHARGE BATTERIES BELOW 9 VOLTS!

ALWAYS UNPLUG BATTERIES AFTER EACH FLIGHT OR TEST!

Propulsion System

A custom engine mount was fabricated for the installation of a Hacker B40-12L motor with a 4.4:1 gearbox.  The motor is controlled by a Castle Creations Phoenix-45 electronic speed control, which is connected to the throttle channel of the radio control (RC) receiver.  As well as providing throttle control the motor controller also uses a battery eliminator circuit (BEC) to provide power to the RC control systems directly from the primary motor battery. The control system is activated via the slide switch of the port side of the fuselage.  The BEC will automatically turn the motor off to conserve power if the battery voltage falls below 9 volts, allowing a gliding landing without loss of control and preventing the lithium polymer packs from being discharged to below 3 volts per cell, or below 9 volts total for the 3-cell packs installed. Do not continue the mission under motor power after low voltage cutoff.  The pilot should initiate a gliding approach and landing, briefly restarting the motor if necessary.

Note that upon connection to the battery, the controller will “play” a series of tones similar to a cell phone ring.  When the motor arm switch and RC transmitter are turned on a second set of tones will play, alerting the operator that the system is fully armed and ready for flight.  At this point be aware the propeller is to be treated with extreme caution!

An APC 15” x 8”E propeller is utilized to give efficient operation with the selected lithium polymer power system.  The propulsion system will run at full throttle for approximately 20 minutes at approximately 300 watts, producing over 4 pounds of static thrust at over 5000 RPM.  The flight time can far exceed 20 minutes, perhaps approaching an hour, by throttling back after achieving a safe altitude.

If necessary, an APC 15” x 10”E propeller may be used to provide additional thrust at the expense of reduced flight times and additional stress on the propulsion system.

KEEP HANDS CLEAR OF PROPELLER AT ALL TIMES!

USE ONLY APC 15”E (ELECTRIC) PROPELLERS!

Radio Control System

ImageAire iii-2 has 2 separate radio control systems, one for flight control and the other for payload control. Both transmitters are completely license free and require no special clearances to operate.

The flight control system is a commercial PCM radio control system by JR (JR XP662).  Control configuration is standard US format with throttle and rudder on the left stick and elevator and ailerons on the right stick.  Please note that the elevator and aileron dual-rate selector switches on the top-left and top-right of the transmitter are NOT active, but may be programmed at the pilot’s discretion.  The rudder and ailerons are mixed electronically with the transmitter to facilitate smoother turns and enhance low speed performance.  The settings for all controls deflections may be modified at the operator’s discretion if deemed necessary.  See the JR radio manual for instructions.

The payload video downlink feed is controlled by a second FM radio control system by Futaba (Futaba SS3).  The downlinked video feed may be switched between the color camera and infrared system by operating the slide switch located on the left-front side of the transmitter.

Either RC transmitter will function for a minimum of 2 hours before needing recharging.  The charge level can be monitored by observing the indicators on the face of the transmitters.

fully extend transmitter antennas DURING operation!

ONBOARD PROPULSION/PAYLOAD POWER SYSTEM

Two packs of lithium polymer batteries wired in parallel are used to provide power for both propulsion and onboard payload power in ImageAire iii-2. A custom inductance-capacitance (LC) filter is installed between the switch and the payload systems to minimize noise from the power system.

Each pack consists of lithium polymer cells arrayed is a 3-parallel/3-series configuration to provide a nominal value of 11.1 volts under load, with a capacity of 6 amp hours each.  These packs are connected in parallel to provide a total of 12 amp hours at 11.1 volts.  This configuration provides an extremely high energy density, but requires some care.  If the battery voltage drops below 3 volts per cell, or 9 volts total for the installed packs, the batteries can sustain permanent damage.  Even with all systems switched off the speed control still draws a small amount of current, which will drain the batteries over time, damaging the batteries.  Always unplug batteries after use.

There are 2 connectors on the battery; a large connector for propulsion and control and a small connector for the cameras and transmitter.  This allows the operator to test/operate the video system without powering the propulsion system, making ground testing safer by preventing accidental motor starts.  Always be sure to disconnect BOTH connectors after each test or flight to prevent battery damage from excessive discharge.

When the battery voltage falls below 9 volts the Castle Creations Phoenix-45 electronic speed control automatically turns the motor off, relieving the current load and alerting the pilot to initiate a gliding approach.  If necessary, the motor can be briefly restarted for a controlled approach and landing without damage to the batteries.

The batteries are accessible by removal of the front hatch.  The batteries are connected to the motor controller harness and payload power connector using the installed polarized connectors.

Normal current levels are 0.5 amps for the payload and control system with the servos quiet, surging to well over an amp with servo movement.  The propulsion system can draw as much as 35 amps under maximum load.

DO NOT DISCHARGE BATTERIES BELOW 9 VOLTS!

ALWAYS UNPLUG BOTH BATTERIES AFTER EACH FLIGHT OR TEST!

BEGIN APPROACH AND LANDING UPON LOW-VOLTAGE CUTOFF!

IMAGING PAYLOAD SYSTEMS

The imaging payload systems are comprised of real-time selectable color and monochrome thermal infrared cameras with video transmission to a ground station. The NTSC format video signal from both cameras feed a switch controlled by the payload RC system, enabling a ground based operator to select the video to be transmitted via a 2.4 GHz transmitter.  This transmitter is FCC Part 15 compliant and requires no special license to operate. Switching between the visible and thermal infrared cameras enables comparison of specific features. The downlink video signal will be captured using a 2.4 GHz video receiver and recorded on a Sony Digital 8mm portable VCR for storage and analysis.

Software, battery chargers, cables, and manuals are provided.

VIDEO SYSTEM GROUND STATION

The ImageAire iii-2 video system ground station consists of a 2.4 GHz video receiver and a Sony Digital 8mm VCR.  After charging batteries, connect the video output of the video receiver to the audio/video input jack on the Sony VCR using the supplied adapter and extension cable.  Turn both units ON, set the video receiver to channel 1 (2401 MHz) using the button on the side of the enclosure, insert tape in VCR, and observe video signal on the VCR LCD monitor.  When the Pilot is ready to launch the aircraft, the Payload Operator should start recording by pressing the RECORD button and the button immediately to the right simultaneously.  Use the payload control RC transmitter to switch video sources as desired while observing the LCD monitor.  When recording is complete, press STOP to halt recording.

Video data may be analyzed post-flight by using a computer with a video capture card to “grab” visible and infrared images of ground features for comparison.

The maximum operating range of this system is approximately 2000 feet, which roughly corresponds to normal line-of-sight radio control flight.  An occasional video signal dropout is to be expected and should not affect data analysis.  If the plane is too far away to get a good video signal it is probably too far away to reliably control.

Battery chargers, cables, and manuals are provided.

BATTERY CHARGING

There are many different batteries in the ImageAire iii-2 system, both airborne and ground based.  All of these batteries are rechargeable and all necessary chargers are provided.

- The JR RC control transmitter is supplied with a factory JR wall charger and is meant to charge overnight (>8 hours) to attain a full charge. There are 2 connectors on the charger.  Use the round connector to charge the transmitter.  The small rectangular connector is not used.  The transmitter power switch should be in the OFF position during charge.

- The Futaba payload control transmitter is supplied with a factory Futaba wall charger and is meant to charge overnight (>8 hours) to attain a full charge. There are 2 connectors on the charger.  Use the round connector to charge the transmitter.  The small rectangular connector is not used.  The transmitter power switch should be in the OFF position during charge.

- The video receiver battery (attached to the receiver) should be charged overnight at the same time as the RC transmitters.  Charge receiver battery by plugging designated wall charger into receiver power socket with battery attached (to bottom of receiver).  Receiver power switch should be in the OFF position during charge.

CAUTIONARY NOTE:  ALTHOUGH TRANSMITTER AND RECEIVER CHARGE PLUGS ARE SIMILAR THEY ARE NOT INTERCHANGEABLE.  IMPROPER CONNECTIONS WILL RESULT IN DAMAGE!  BE SURE TO USE THE PROPERLY LABELLED CHARGER FOR EACH DEVICE!

- The Sony portable VCR has an internal charger for its lithium battery.  The battery should be attached to the VCR.  With VCR power off, the charge light will indicate that the battery is charging.  A full charge can take as long as 8 hours.  A single charge should be adequate to record several flights.

- The onboard batteries are two (2) lithium polymer 11.1v/6ah battery packs connected in parallel that are charged using the supplied Kokam LIPO-402 charger, powered using the provided 12 VDC power supply.  Plug the output of the power supply into the charger power cord.  Please charge as described in following procedure.

ONBOARD PROPULSION/PAYLOAD BATTERY

Charging procedure

- Verify charger is on and the jumpers are set for 1.5 amps and 11.1 volts.  Verify batteries are close to room temperature (not hot).

- Connect batteries to charge cord via large red/black polarized connectors.  Mode indicator light/lights should come on.

- Battery will begin to charge.  Battery voltage will begin to rise.

- The green full light on the LIPO-402 charger will light when the charge is completed.

- Completely depleted battery can take as long as 10 hours to charge, much less if only partially discharged.

ALLOW BATTERIES TO COOL BEFORE CHARGING!

ALLOW BATTERIES TO COOL BEFORE FLIGHT!

HAPPY FLYING!

For further information or questions contact:

Ted Miles: 757-824-2937, ted.k.miles@nasa.gov

Geoff Bland: 747-824-2855, geoff.bland@nasa.gov

ImageAire iii-2 System Layout