Control Station for MLB BAT 3

Primary control is through GPS-guided flight to predetermined waypoints. The waypoints and flightplans are created in the MLB groundstation software, used for communicating with the BAT for flight planning, initializing the launch sequence, pre-launch programming, in-flight retasking and display of live telemetry. A moving map display shows the aircraft’s position, waypoints and flight path. Aircraft state information that is visible in real-time on the ground station includes battery voltage, waypoint location, GPS status, position, range, airspeed, groundspeed, heading, altitude, wind speed, wind direction, RPM, temperature, fuel gauge.

One control station is used.

The secondary control is manually with a radio control transmitter (72 MHz R/C 8-channel PCM uplink). Within a distance of 1-2 miles, the aircraft can be switched from autonomous to manually anytime. A main and a backup radio control transmitter are used.

The BAT 3 is launched from a catapult on top of a vehicle. After launch, the first waypoint is acquired and the flight continues GPS guided. Landing can be performed either autonomously or manually. Manual landings are preferred for closer control over landing location (due to some GPS error of autonomous landings). During launch and autonomous landings, a pilot stands by with the RC control unit, ready to switch to manual in case of problems.